Creative Technology / Amstrad PLC

ROSANNE

Resident O.S. for PcW16 ('Anne')

d2.83 16-August-96

All functions described herein are entirely fictional. Any resemblance to actual operating systems, whether living or dead, is entirely coincidental.

Memory Layout

ROM blocks numbered #00...#3F, RAM blocks numbered #80...#BF

Application programs ...	WP to be in ROM blocks #0C..#11

SS to be in ROM blocks #12..#13

AB to be in ROM blocks #14..#15

DA to be in ROM block #16

CC to be in ROM block #17

FM to be in ROM blocks #20..#21

These block numbers are given as constants rom_wp, rom_ss etc. in romdefs.inc (\PSYQ\INCLUDE).

The appropriate starting block will switched in to 2nd quarter, with initial execution address = #4100 (for historical reasons - you can put what you want in the first 256 bytes of your first ROM). Use your own variables for first_ROM_blk so that your program can be moved around later! (Apps will be stored in contiguous blocks). Switching in further blocks of your app is up to you (you can do what you want with the 2nd, 3rd and 4th quarters of memory).

RAM can be got from the OS in 16k lumps. This will NOT necessarily be contiguous. You should NEVER use RAM which you haven't officially got from the OS. You must also remember to release RAM when you've done with it ...

RAM block #80 is in 1st quarter (#0000...#3FFF) and should stay there! AP may use #2000...#2FFF of this as its own. Stack descends from #3FFE (i.e. top of block #80) and has about 2k space (that should be enough!!!). AP may alter contents of other three quarters at will. Don't try to use bios_setmem0, or to switch any bank other than #80 into #0000...#3FFF directly.

Interrupts

Interrupts must be disabled / re-enabled through bios_di & bios_ei (because of the kbd cockup).

If you leave interrupts disabled for long periods the debugger will report 'Target does not respond' as the communication with the Windows m/c is part of the interrupt handler! This is non-fatal though, and the report will clear itself automatically when communication is restored.

RST #10   (mcall macro)

Allows code to be executed with automatic mousoff / mouson around it.

Put	mcall	code_address	to execute 	CALL bios_mousoff

CALL code_address

CALL bios_mouson

All registers are passed to code_address intact except SP (1 extra word is pushed onto the stack).

Creates 3 bytes of code for each invocation.

RST #30  (farcall macro)

Allows code to be executed in a block not currently switched into Z80's 64k memory space.

Put farcall block,address to execute code at address in block. The top two bits of address are used to determine which quarter of the Z80's map to switch block into.

All registers are passed to address intact except SP (3 extra words pushed onto the stack).

Creates 4 bytes of code for each invocation.

Addressing

16-bit addresses are read by the OS as being within the current bank setup.

16-bit addresses RETURNED by the OS are always in the bottom qtr (block #80)

24-bit addresses (usually in CHL or BDE) are read by the OS as an 8-bit bank no (C/B) and a 14-bit address (HL/DE). Bank no = #00..#7F for ROM, or #80..#FF for RAM. The top two bits of any register holding the 14-bit address are ignored, except in the case of the bios_intserve function.

The Keyboard

The keyboard generates two bytes for each keystroke: an 'ASCII' code byte and a bit-significant Shift-State byte. When this happens, the OS generates a 'keyboard-code-received' message (see os_get_event below) and these 2 bytes are then available to the programmer via the os_get_event function (see below).

Because of the requirements of the Drop-Down-Menu system, the programmer may NOT re-define the keyboard ... the previously-provided bios_defkey function has been removed! The keyboard has been pre-defined to meet the requirements of the typography system, so that:

*	All key-combinations which generate a printable character generate the appropriate code for that character (e.g. Shift+Ctrl+K produces a code #84 = closed bullet).

*	Any alpha-key combination which does NOT produce a printable character is set to produce the same code byte regardless of the state of Shift, Alt and Ctrl (e.g. Ctrl+'b' gives the same ascii code as 'b' alone, but you can always tell that it was a Ctrl+'b' as opposed to a 'b' from the Shift-State byte).

*	The 'action' keys (TAB, Backspace, Return/Enter, Cursor U D L R, Insert, Delete, Home, End, PgUp, PgDn, Stop and Function strip) generate their own unique code bytes, but the code for each is the same regardless of the Shift-State.

*	The keypad keys mostly generate the same code bytes as their counterparts in the main keyboard, except that a 'Keypad' bit (b4) is set in the Shift-State byte which informs you that the key pressed was one of the keypad group. This allows you to distinguish between e.g. Return & Enter, or between the two % keys.  N.B. The keypad DIVIDE, MINUS and TIMES are NOT copies of the / - and * keys on the main keyboard, and generate different code bytes.

The code bytes are documented in full in Appendix A.

The Shift-State byte is bit-significant as follows:

Bit0:	  Shift

Bit1:	  Alt

Bit2:	  Ctrl

Bit3:	  reserved

Bit4:	  Keypad

Bits567: reserved

The Recovery Jumpblock

For recovery-from-disaster and OS-upgrading there is a jumpblock of critical low-level functions in ROM0, starting at address #4006, with each entry taking three bytes. The entries are ...

#4006	jp rom0_loadsector

#4009	ICP-ed version number (6 bytes ... e.g. 'd2.50',0)

#400F	jp rom0_loadsector_init

#4012	jp recover_os

#4015	jp cl_picture

#4018	jp warm_start

#401B	jp bmp_pointer

Execution Protocols (do you want a blindfold? any last requests?)

When an app is run execution starts at #4100 with CHL = 24-bit address of i/o buffer (as per os_call_task)

on input ... 	buffer	+0	Word	Command Length (incl. Command Number) in bytes

+2	Byte	Command Number

+3...		Command Parameters

on output* ...	buffer	+0	Word	Output Data Length in bytes

+2...		Output Data

*n.b. some commands have no output, others do not return at all !

Command Numbers, Apps to which each applies, Parameters & Output:

No.		Apps	Meaning			Input Params			Output

xp_end	ALL	close all ram usage & end	none				none

xp_cold	ALL	cold start as if no previous job	none				none - no return

xp_ continue	ALL	continue previous job		none				none - no return

xp_new	WP SS	start new thang*		none				none

xp_edit	WP SS	edit existing thang*		none				none

xp_print	WP SS	print existing thang*		none				none

xp_calc	CC	return caculation		+3...o/p address (Word24)	ascii result

+6...previous appid (Word)

xp_abrec	AB	get whole record		+3...16-bit appt ID (Word)	record data

+6...o/p address (Word24)

xp_abID	AB	user-select one entry		+3...o/p address (Word24)	record ID

xp_abIDlist	AB	user-select several entries	+3...o/p address (Word24)	record IDs

xp_dapam	DA	post alarm maintenance		+3...appt ID (Word)		none, but

+5...ack/timeout flag		error flag in A

+6...foreground flag		(0=OK / non-0=err)

xp_wpinit	WP	make user template		+3...User Info addr (Word24)	none, but

error flag in A

(0=OK / non-0=err)

*thang = document, sheet, etc.

These xp_ command number constants are declared in CONSTOS.Z80.

N.B. Exiting applications:

* Apps should exit from xp_end, xp_cold, xp_continue, xp_new, xp_edit, xp_print by os_appexit.

* Apps should exit from xp_calc, xp_abrec, xp_abid, xp_abidlist, xp_dapam, xp_wpinit by a simple RETURN, leaving the 3rd and 4th quarters of memory as they were on entry.

OS functions

All accessed using CALL #003B, with the OS function number in A.

The os_xxxx function numbers are defined as constants in the file = CONSTOS.Z80

The  management accepts no liability for the nervous breakdowns of programmers who hard-code function numbers etc. into their source !!!!

n.b. things marked @ don't exist yet !!!!

things marked > are new since d2.zz, things marked >> are very new !

EVENT HANDLING ...

No.	Name

#	os_get_event		get selected event from OS event pipe (if any)

in:	HL = Event Mask (ALWAYS #FFFF)

out:	Z if no matching event pending... IX & AF corrupt, all other registers preserved

NZ if there is one .....................	HL = Event Code (any one bit set only)

IX  = 16-bit address of 12-byte Event record

AF, BC, DE corrupt, all other registers preserved

Any events pending which do NOT match the mask will be discarded from the pipe so NEVER

use anything other than #FFFF as the mask!

Event Code/Mask as follows:

bit 0	Timer

bit 1	DDM selection

bit 2	Right Mouse Button Single-Click

bit 3	Right Mouse Button Double-Click

bit 4	Right Mouse Button Release

bit 5	Centre Mouse Button Single-Click

bit 6	Centre Mouse Button Double-Click

bit 7	Centre Mouse Button Release

bit 8	Left Mouse Button Single-Click

bit 9	Left Mouse Button Double-Click

bit 10	Left Mouse Button Release

bit 11	Mouse Movement

bit 12	Mouse Entered/Exited Rectangle

bit 13	OS message

bit 14	Character from Keyboard

bit 15

Event record (16-bit address of 12-byte record returned in IX) as follows:

Mouse/Key Events:

IX+0	Word	Event Code (as above)

IX+2	Byte	Key: ASCII code				  	Mouse: unused

IX+3	Byte	Key: Shift State	bits 4,2,1,0 = Kypd,Ctrl,Alt,Shift		Mouse: unused

IX+4	Word	unused

IX+6	Word	Mouse x

IX+8	Word	Mouse y

IX+10	Byte	Buttons		bits 2,1,0 = L,C,R (1=pressed)

IX+11	Byte	Shift State	bits 4,2,1,0 = Kypd,Ctrl,Alt,Shift

Kypd=1 means the key was a Keypad key

Mouse Enter/Exit Rectangle Events:

IX+0	Word	Event Code (as above)

IX+2	Byte	0=Enter / 1=Exit

IX+3	Byte	unused

IX+4	Word	unused

IX+6 ...	as for Mouse/Key events

Timer Events:

IX+0	Word	Event Code (as above)

IX+2	Byte	Timer Number		0..3

IX+3	Byte	Repeat Mode		0=oneshot / 1=repeating

IX+4	Word	Interval in 1/16th secs	0..57599 (1 hour)

IX+6 ...	as for Mouse/Key events

DDM Selection Events:

IX+0	Word	Event Code (as above)

IX+2	Byte	Title Number #00..#0A,  #FE for Undo,  #FF for Help

IX+3	Byte	Entry Number #00..#1F

IX+4	Byte	New Status of Entry (see os_ddm_define below)

IX+5	Byte	unused

IX+6 ...	as for Mouse/Key events

OS Message Events:

IX+0	Word	Event Code (as above)

IX+2	Byte	Message ID (see below)

IX+3	Byte	Message Data Byte

IX+4...		Message Data Bytes/Words

Message IDs	Data Byte / Words			Action

om_topbar	Byte	Title Number			pass to os_process_event

om_exit	none					close all ram and os_app_exit

om_objevent	Byte	Object Number			pass to os_process_event

Byte	Object Type			           (optionally!)

Byte	Object Event Type ...

@		   obev_msin		mouse moved into object

@		   obev_msout		mouse moved out of object

obev_click		mouse-click on object

(IX+10 = non-0 for no-hilite-focus) *

obev_dclick		mouse-double-click on object

obev_key		object key pressed (txt/bmpbutton only)

obev_curup		up key pressed on focussed object

obev_curdn		down key pressed on focussed object

obev_curhome	home key pressed on focussed object

obev_curend		end key pressed on focussed object

obev_curpgup	pageup key pressed on focussed object

obev_curpgdn	pagedn key pressed on focussed object

obev_focfire		focussed object 'fired' (space-bar)

obev_clickout		mouse clicked outside dialog area

obev_focup		focus to next object

obev_focdn		focus to previous object

Passing om_objevent messages to os_process_event records the value changes in the dialog data (except scroll-bars / spinners) and returns value data - see below

* If an obev_click is on an ob_textedit and (IX+10) is non-0, passing the event to os_process_event will put it into edit mode with NO highlight.

No.	Name

#	os_wait_event		wait for specified event(s) from OS event pipe

in:	HL = mask of events to wait for (must be #FFFF)

out:	as os_get_event (but can't be Z for no event!!)

#	os_put_event		put an event into the OS event pipe

in:	C,HL = 24-bit address of 12-byte event record

out:	NC/C ..	OK / Error in event record

#	os_process_event	pass event on to OS for processing

N.B.!		in:	IX = 16-bit address of 12-byte event record ( passed on from os_get_event ! )

out:	om_topbar	A = 0/non-0	didn't / did generate a DDM selection event

om_objevent:

obev_click/_dclick/_curup/_curdn/_focfire/_key:

CHL = addr of copy of Object Record (see os_dialog_define)

BDE = extra info for certain object types only as follows:

ob_textedit:	    B =	exit code (as os_text_edit)

ob_v/hscroll:	    B =	1	scrollbar up(left) arrow clicked

2	scrollbar page up(left) clicked

3	scrollbar handle move ... DE=new posn

4	scrollbar page down(right) clicked

5	scrollbar down(right) arrow clicked

ob_spinner:	    B =	1	spinner up

5	spinner down

#	os_flush_events		flush all pending events from the OS event pipe

in:	NONE

out:	NONE. AF, HL corrupt

#	os_def_evrect		define rectangle for mouse enter/exit events

in:	C,HL = 24-bit address of x, y, w, h (pixels relative to screen top-left)

out:	C .......	rectangle is wholly/partly off screen

NC .....	OK

#	os_get_evrect		get position of rectangle for mouse enter/exit events

in:	NONE

out:	HL = 16-bit address of x, y, w, h

#	os_set_timer		set up a timer event

in:	C,HL = 24-bit address of Timer Record (see os_get_event, but only bytes 2..5 needed)

out:	NC/C ..	OK / Error in Timer Record

#	os_kill_timer		remove a timer event

in:	E = timer number 0..3

out:	NC/C ..	OK / illegal timer number

No.	Name

#	os_set_delayed_event	set up a delayed farcall event

in:	DE = number of seconds delay (+/- 1 second error in worst case)

CHL = farcall address to be executed during first os_get_event after delay period expires

(top two bits of HL ARE significant as per farcall macro - see above)

N.B. The code at CHL must RET as normal, preserving ALL registers except AF.

out:	NONE. AF corrupt

#	os_kill_delayed_event	kill any existing delayed farcall event

in:	NONE

out:	NONE. AF,HL corrupt

THE DROP-DOWN-MENU SYSTEM (DDM) ...

No.	Name

#	os_ddm_define	define the DDM & display the title-bar

in:	C,HL = 24-bit address of DDM definition.

B  = Undo / Help flags	Bit 3 = include Undo

Bit 2 = make Undo grey

Bit 1 = include Help

Bit 0 = make Help grey

DDM definition is sequential data for each menu, arranged ...

Title 0, Entry 0, Entry 1, ..., Entry n (0-terminated)

Title 1, Entry 0, Entry 1, ..., Entry n (0-terminated)

Title 2, Entry 0, Entry 1, ..., Entry n (0-terminated)

...	  ...	  ...	...    ...		...

Title n, Entry 0, Entry 1, ..., Entry n (0-terminated)

Where each line of the above is made up as follows ...

Title Definition ...

+0	Byte:	Title Status ...	Bit 7 = always 1

Bits 6,5,4 unused at present

Bit 3 = greyed if 1

Bits 2,1,0 unused at present

+1	Word	unused

+3...	ASCII	Title String	7-bit ASCII data, terminated with |

ul_code (#1F) before a char = underlined selector key

Each Entry Definition ...

+0	Byte:	Entry Status ...	Bit 7 = always 1

Bits 6,5,4 unused at present

Bit 3 = greyed if 1

Bit 2 = ticked if 1

Bit 1 = tickable if 1

Bit 0 unused at present

+1	Word	Hotkey		Byte0 = ASCII

Byte1 = Shift State

+3...	ASCII	Entry String	ASCII data, |-terminated ... except last 0-terminated

ul_code (#1F) before a char = underlined selector key

For a Divider Line ...

+0	Byte:	Divider Status	#80

+1	ASCII	Divider String	'_','|'  	(or '_',#00 if last entry)

If no entries are defined for a particular title, just put a single #00 byte as the first entry

Whole DDM definition is terminated with a final #00 in place of the next title definition

'Hotkeys' are in addition to the (possible) underlined keys defined in the ASCII strings.

>			Max titles = 10, Max entries per title = 20, Max total length of all entry data = 2048 bytes

out:	NONE

AF, BC, DE, HL, IX, IY corrupt

Note:	The OS copies the data, so RAM pointed to by CHL can be released after this function

#	os_ddm_display		display the DDM title-bar

in:	NONE

out:	AF, BC, DE, HL, IX, IY corrupt

No.	Name

#	os_ddm_get_tasks		get the current definition of the Tasks menu

in:	C,HL = 24-bit address of buffer to put Tasks menu definition data in

out:	HL = 14-bit address of next byte after tasks list (in block defined by C)

i.e. CHL points to where to add your own data for other menus !

AF, BC, DE corrupt

#	os_ddm_get_title_status	get status of a title in the DDM

in:	C = title 0..9  or  #FE / #FF for Undo / Help

out:	C .......	illegal title no

NC .....	A = title status (as above).  For Undo / Help, OFF/GREY/ON = #00/#88/#80

AF, BC, IY corrupt

#	os_ddm_put_title_status	change status of a title in the DDM

in:	C = title 0..9  or  #FE / #FF for Undo / Help

E = title status (as above).  For Undo / Help, OFF/GREY/ON = #00/#88/#80

out:	C .......	illegal title no

NC .....	OK

AF, BC, IY corrupt

#	os_ddm_get_entry_status	get status of an entry in the DDM

in:	C = title 0..9

B = entry 0..19

out:	C .......	illegal title / entry no

NC .....	A = entry status (as definition above)

BC, IY corrupt

#	os_ddm_put_entry_status	change status of an entry in the DDM

in:	C = title 0..9

B = entry 0..19

E = entry status (as definition above)

out:	C .......	illegal title / entry no

NC .....	OK

AF, BC, IY corrupt

#	os_ddm_disable		disable DDM

in:	NONE

out:	NONE. AF corrupt

#	os_ddm_enable		re-enable DDM after os_ddm_disable has been used

in:	NONE

out:	NONE. AF corrupt

#	os_ddm_getenable	get current disabled/enabled status of DDM

in:	NONE

out:	A = 0/255 = disabled/enabled

THE PROGRAM MANAGER ...

No.	Name

#	os_get_version		get OS version identifier

in:	none

out:	CHL = 24-bit address of version string (6 bytes including terminating 0)

#	os_app_exit		exit current app (return to desktop)

in:	none

out:	no exit!

#	os_run_program		run named external program

in:

out:	no exit!

#	os_wp_usertemplate	make user template from User Information

in:	CHL = 24-bit address of User Information (in AB record format)

out:	A = 0	OK

A = 1	Could not make template

#	os_calculator		call calculator (for 'Insert Calculation' functions)

in:	CHL = 24-bit address of Result Buffer	- at least 19 bytes, NOT in range #802000..#802FFF

>>		If the buffer is pre-loaded starting #AA, #55 followed by a 0 terminated string then that string

>>		will appear on the Exit button under "Exit to the" instead of "Calling Program".

out:	Result Buffer = 0-terminated output string (the display in ASCII including commas)

#	os_ab_get_record	get data for a particular record from Address Book

in:	DE = record ID

CHL = 24-bit address of Result Buffer	- at least 1500 bytes, NOT in range #802000..#802FFF

out:	A = 0	Result Buffer = length, record data

A = 1	ID out of range

A = 2	Could not open AB (not enough memory / file error)

#	os_ab_select_ID		allow user-selection of a single record ID from Address Book

in:	CHL = 24-bit address of Result Buffer	- at least 4 bytes, NOT in range #802000..#802FFF

out:	A = 0	Result Buffer = length(2), selected ID (word)

A = 1	User CANCEL-ed

A = 2	Could not open AB (not enough memory / file error)

#	os_ab_select_IDlist	allow user-selection of multiple record IDs from Address Book

in:	CHL = 24-bit address of Result Buffer	- at least 1822 bytes, NOT in range #802000..#802FFF

out:	A = 0	Result Buffer = length, list of selected IDs (words)

A = 1	User CANCEL-ed

A = 2	Could not open AB (not enough memory / file error)

#	os_da_postalarm	post-alarm maintenance call to Diary/Alarm app

in:	DE = appointment ID of alarm

B = 0 / non-0 = acknowledged by user / timed out

out:	A = 0 / non-0 = OK / Could not open DA appointments file

THE MEMORY ALLOCATION SYSTEM ...

No.	Name

#	os_open_folder		open a folder

in:	NONE

out:	A = folder_id #80-8F or 0 if no folder available

F corrupt

#	os_close_folder		close a folder, releasing all attached RAM

in:	E = folder_id #80-8F

out:	C .......	folder_id illegal.

AF corrupt

NC ....	A = no of 16ks free AFTER release

B = no of 16ks owned by applications AFTER release

C = no of 16ks owned by folders AFTER release

F corrupt

#	os_ask_mem		ask how much free RAM remains

in:	NONE

out:	A = no of 16ks free

B = no of 16ks owned by applications (incl. current_app)

C = no of 16ks owned by folders

F corrupt

#	os_get_mem		get RAM from OS

in:	B = no of 16ks required

E = owner_code #00 / #80-8F = RAM to be owned by current_app / folder #80-8F

out:	C ........	owner_code illegal.

AF, DE, HL corrupt

NC ......	B = no of 16ks allocated (may be less than requested if not enough available)

HL = 16-bit address of list of block numbers, 0-terminated

AF, C corrupt

#	os_release_mem	release RAM back to OS

in:	C,HL = 24-bit address of list of block numbers, 0-terminated

E = owner_code #00 / #80-8F = RAM to be released from current_app / folder #80-8F

out:	C ........	owner_code illegal.

AF, DE, HL corrupt

NC ......	A = no of 16ks free AFTER release

B = no of 16ks owned by applications AFTER release

C = no of 16ks owned by folders AFTER release

F, DE, HL, IX corrupt

#	os_release_allmem	release all RAM attached to an application / folder

in:	DE = 16-bit ID 	#008x = release all RAM from folder #8x

#xxxx = release all RAM from application #xxxx

out:	A = no of 16ks free AFTER release

B = no of 16ks owned by applications AFTER release

C = no of 16ks owned by folders AFTER release

F, DE, HL corrupt

No.	Name

#	os_get_memlist		get list of 16ks owned by an application / folder

in:	DE = 16-bit ID 	#008x = folder #8x

#xxxx = application #xxxx

out:	B = no of 16ks owned by application / folder

HL = 16-bit address of list of 16ks (0-terminated) ... order IS significant

AF, C corrupt

#	os_get_mem_ownerid	ask ID of application / folder which owns a 16k RAM block

in:	E = 16k RAM block no #80..#FF

out:	C .......	RAM block no illegal.

AF corrupt

NC ....	DE = app_id / folder_id which owns 16k block ...

#0000 = block free

#008x = block owned by folder #8x

#00xx  = block owned by OS

#01xx  = block owned by application #xxxx

AF, HL corrupt

#	os_get_folder_appid	ask ID of application which opened a folder

in:	E = folder_id #80-8F

out:	C .......	folder_id illegal.

AF corrupt

NC ....	DE = app_id of application which opened folder_id (#0000 if folder_id not open)

AF, HL corrupt

#	os_get_next_mem	get next block of memory

in:	E = current block number

out:	C .......	A=0 ......... no next block

A=1 ......... current block is unowned

NC ....	E = next block number

F corrupt

#	os_copy_24bit	copy from 24-bit source to 24-bit destination with block roll-over

in:	C,HL = 24-bit source	(data may continue into next block(s) owned by owner of C)

B,DE = 24-bit dest	(data may continue into next block(s) owned by owner of B)

IX = 16-bit length

out:	NC/C ..	OK / Error (no further source/destination block to roll-over into)

C,HL = next address to copy from (C=#00 if no more source blocks)

B,DE = next address to copy to (B=#00 if no more destination blocks)

IX = amount uncopied (0 if OK)

A corrupt

No.	Name

#	os_get_byte_24bit	get byte from 24-bit source

in:	C,HL = 24-bit address

B = wind-on flag    0 = leave CHL as is,  1 = wind CHL on by 1

out:	E = byte value

C,HL = unaffected / wound-on address (C=#00 if no more source blocks)

A corrupt

#	os_get_word_24bit	get word from 24-bit source

in:	C,HL = 24-bit address

B = wind-on flag    0 = leave CHL as is / 1 = wind CHL on by 2

out:	DE = word value

C,HL = unaffected / wound-on address (C=#00 if no more source blocks)

A corrupt

#	os_get_wd24_24bit	get wd24 from 24-bit source

in:	C,HL = 24-bit address

B = wind-on flag    0 = leave CHL as is / 1 = wind CHL on by 3

out:	BDE = wd24 value

C,HL = unaffected / wound-on address (C=#00 if no more source blocks)

A corrupt

#	os_put_byte_24bit	get byte from 24-bit source

in:	C,HL = 24-bit address

B = wind-on flag    0 = leave CHL as is,  1 = wind CHL on by 1

E = byte value

out:	C,HL = unaffected / wound-on address (C=#00 if no more source blocks)

A corrupt

#	os_put_word_24bit	get word from 24-bit source

in:	C,HL = 24-bit address

B = wind-on flag    0 = leave CHL as is / 1 = wind CHL on by 2

DE = word value

out:	C,HL = unaffected / wound-on address (C=#00 if no more source blocks)

A corrupt

#	os_put_wd24_24bit	get wd24 from 24-bit source

in:	C,HL = 24-bit address

B = wind-on flag    0 = leave CHL as is / 1 = wind CHL on by 3

IX(lo),DE = wd24 value

out:	C,HL = unaffected / wound-on address (C=#00 if no more source blocks)

A corrupt

#	os_add_24bit		add to a 24-bit address

in:	C,HL = 24-bit address

BDE = true-24-bit amount to add (top two bits of DE ARE meaningful)

out:	NC/C ..	OK / Error (no further block to roll CHL into)

C,HL = updated address (C=#00 if no more source blocks)

A corrupt

THE HIDEOUS RACKET SYSTEM ...

No.	Name

#	os_click		click (!)

in:	none

out:	none

AF corrupt

#	os_beep		beep (!)

in:	none

out:	none

AF corrupt

THE TIME & DATE SYSTEM ...

No.	Name

#	os_get_timedate		get system time & date

in:	NONE

out:	HL = 16-bit address of 8-byte Time & Date record in bottom quarter (block #80) ...

+ 00	Byte	Ticks		(0...255)

+ 01	Byte	Seconds	(0...59)

+ 02	Byte	Minutes		(0...59)

+ 03	Byte	Hours		(0...23)

+ 04	Byte	Day		(1...31)

+ 05	Byte	Month		(1...12)

+ 06	Word	Year		(0...9999)

AF, BC, DE corrupt

#	os_set_timedate		set system time & date

in:	C,HL = 24-bit address of 8-byte Time & Date record

out:	C .......	Error in Time & Date record

NC .....	OK

AF, BC, DE, HL corrupt

#	os_get_time_offset	get system time offset from GMT/UST

in:	NONE

out:	HL = 16-bit address of 4-byte Time Offset record ...

+ 00	Byte	unused

+ 01	Byte	Seconds	(-59...+59)

+ 02	Byte	Minutes		(-59...+59)

+ 03	Byte	Hours		(-23...+23)

AF, BC, DE corrupt

#	os_set_time_offset	set system time offset from GMT/UST

in:	C,HL = 24-bit address of 4-byte Time Offset record (see os_get_time_offset)

out:	C .......	Error in Time Offset record

NC .....	OK

AF, BC, DE, HL corrupt

#	os_date_to_ascii	convert date to ASCII

in:	C,HL = 24-bit address of 8-byte Time & Date record (Time fields ignored)

B = format ......	bit     0	= day .............	0 / 1		=  plain / ordinal

bits 2,1	= month .........	00 / 01 / 10 / 11	=  num / 3-letter / full-name / ---

bit     3	= year ............	0 / 1		=  2-digit / 4-digit

bit     4	= order ...........	0 / 1		=  d-m-y / m-d-y

bit     5  = leading 0s ...	0 / 1		=  omit / include  (d & m only)

bits 7,6	= separator .....	00 / 01 / 10 / 11	=  spc / slash / hyphen / colon

e.g.	15/03/97 ..........	B = % 01  1  0  0  00  0

3:15:1997 ........	B = % 11  0  1  1  00  0

Mar-03-1997 ....	B = % 10  1  1  1  01  0

15th March 97..	B = % 00  0  0  0  10  1

out:	C .......	Error in Date field(s) of Time & Date record

NC .....	HL = 16-bit address of ASCII string:  + 00    String length (NOT incl this byte)

+ 01... ASCII

AF, BC, DE, IX, IY corrupt

No.	Name

#	os_time_to_ascii	convert time to ASCII

in:	C,HL = 24-bit address of 8-byte Time & Date record (Date fields ignored)

B = format ......	bit     0	= seconds ......	0 / 1		=  omit / include

bit     1  = am-pm ........	0 / 1		=  omit / include

bit     2	= 24hr ............	0 / 1		=  12hr / 24hr

bit     3	= unused

bit     4	= unused

bit     5 =  leading 0s ...	0 / 1		=  omit / include  (hours only)

bits 7,6	= separator .....	00 / 01 / 10 / 11	=  spc / fullstop / hyphen / colon

e.g.	19:02:27 ..........	B = % 11  1  xx  1  0  1

7:02pm ............	B = % 11  0  xx  0  1  0

19.02 ..............	B = % 01  1  xx  1  0  0

out:	C .......	Error in Date field(s) of Time & Date record

NC .....	HL = 16-bit address of ASCII string

AF, BC, DE, IX, IY corrupt

#	os_get_weekday		get day-of-week

in:	C,HL = 24-bit address of 8-byte Time & Date record (Time fields ignored)

out:	C .......	Error in Date field(s) of Time & Date record

NC .....	A = 0..6 for Mon..Sun

HL = 16-bit address of ASCII string

F, BC, DE, IX corrupt

#	os_cp_timedates	compare two times & dates

in:	C,HL = 24-bit address of FIRST 8-byte Time & Date record

B,DE = 24-bit address of SECOND 8-byte Time & Date record

out:	C .......	Error in Time & Date record(s)

NC .....	A = 0  if  FIRST and SECOND equal

1  if  FIRST is later than SECOND

2  if  SECOND is later than FIRST

F, BC, DE, HL corrupt

#	os_get_days_in_month	get number of days in a month

in:	C = month 1..12

DE = year

out:	C .......	Error in month/year

NC .....	A = number of days 28..31

BC, DE corrupt

No.	Name

#	os_clock_full		display system clock, full hr:mn:sc

in:	DE,HL = clock x,y (top-left of 1st hours digit)

B = 0/non-0 for white/black background

out:	C .......	Clock falls off screen

NC .....	OK

#	os_clock_secs		display system clock, seconds only

in:	DE,HL = clock x,y (top-left of 1st hours digit)

B = 0/non-0 for white/black background

out:	C .......	Clock falls off screen

NC .....	OK

#	os_set_alarm		set the system alarm

in:	DE = Appointment ID

C,HL = 24-bit address of Appointment record ...

+0	Byte	Class - 	Bit 0		0/1 = Silent / Audible

Bit 1		0/1 = No wake-up / wake-up if m/c in sleep mode

+1	Byte	Minutes

+2	Byte	Hours

+3	Byte	Day

+4	Byte	Month

+5	Word	Year

+7	Word	unused

+9 ...	ASCII	0-terminated text of alarm message (max 64 bytes)

out:	C .......	Error in Appointment Record

NC .....	OK

AF corrupt

#	os_get_alarm		get current alarm setting

in:	CHL = 24-bit address of buffer to fill with Appointment Record (must be at least 75 bytes)

out:	C .......	No alarm set

NC .....	DE = Appointment ID

Buffer filled with current alarm Appointment Record (see os_set_alarm)

AF corrupt

THE IDLE-TIMER ...

No.	Name

#	os_idle_reset		reset the idle-timer

in:	none

out:	none

AF corrupt

#	os_idle_suspend	suspend the idle-timer (until next os_idle_reset)

in:	none

out:	none

AF corrupt

@	os_idle_do		go into idler mode (screen-saver)

in:	none

out:	none - does not return until a mouse/keyboard event received

AF corrupt

THE DIALOG SYSTEM ...

No.	Name

#	os_dialog_define	define a dialog

in:	C,HL = 24-bit address of dialog definition (max length 16k) :

Global Definition:

+0		Byte	focus		0 = no focus / non-0 = focus exists

+1		Byte	exit-by-click	0 = no exit / non-0 = exit on click outside dialog

+2		Byte	screen-store	0 = no store & restore / non-0 = do

+3		Byte	ddm_enable	0 = no ddm during dialog / non-0 = allow ddm

Box&Title Definition:

+0		Byte	dg_dialog_box / dg_greybutton / 0 if no box

+1		Words	x,y,w,h		global x,y = origin for object coords (even if no box)

+9..		Ascii	Title Text, #00-terminated

Object definition 1:

+0		Byte	object_type	ob_textbutton / ob_textedit / ob_vscroll / etc

+1,3,5,7	Words	x,y,w,h		x,y relative to x,y in Box&Title Definition

+9..	 according to object_type as follows ...

ob_box:

+9...		Byte	linked object number (0 if none)

+10,11,12,13	Bytes	as dg_multi_box + 9,10,11,12 (os_scr_direct)

ob_label:

+9...		Byte	linked object number (0 if none)

+10		Byte	font number

+11		Byte	alignment 0/1/2 = L/C/R

+12...		Ascii	Text, 0-terminated (max 128 chars)

ob_textbutton:

+9,11,13,15	Words	Keycodes ( 4 pairs of Ascii, Shift[#80=ignore] )

+17		Byte	Repeat Flag ... 0 = no repeat / 1 = repeat

+18...		Ascii	Text, 0-terminated --- max 33 bytes incl 0

ob_bmpbutton:

+9,11,13,15	Words	Keycodes ( 4 pairs of Ascii, Shift[#80=ignore] )

+17		Byte	Repeat Flag ... 0 = no repeat / 1 = repeat

+18,20,22,24	Words	dx,dy,w,h

+26		Wd24	Bitmap data address

If bank (+28) = 0, address (+26,7) = pre-defined

graphic no .....	0 = up-arrow	(w=17 h=17)

1 = down-arrow	(w=17 h=17)

2 = left-arrow	(w=17 h=17)

3 = right-arrow	(w=17 h=17)

>									4 = stop-sign	(w=39 h=39)

ob_checkbox:

+9			Byte	Checked status	#00 / #FF = un- / checked

ob_radio:

+9			Byte	Status	#00 / #FF = un- / selected

+10		Byte	Radio Number 0...15

ob_textedit:

+9...	32-byte	Params as os_text_edit, but only need to set fields ***

+0..15		mode..XYWH		set by OS

+16	Word	dialog font no 1..12			***

+18..20				reserved

+21	Wd24	addr of legal chars			***

+24	Byte	length of legal chars			***

+25	Wd24	o/p address		set by OS

+28	Word    max length (max 128)			***

+30..31				set by OS

+41...	Ascii	Current Text, 0-terminated		***

No.	Name

#	os_dialog_define continued

ob_list:

+9		Byte	linked (vscroll) object number (0 if none)

+10	Word	Number of list entries (Ntot)

+12	Word	Number of visible list entries (Nvis)

+14	Word	First visible entry (0..Ntot-1)

+16	Word	Reserved - previous first visible

+18	Word	Current list selection

+20	Word	Reserved - previous list selection

+22	Byte	Dialog Font Number (1..12)

+23	Word	Height of each entry

+25	Wd24	24-bit address of string-list = list of 24-bit addresses

(strings may include tabs)

+28	Word	Tab position (in pixels from x)

ob_listmulti:

+9		Byte	linked (vscroll) object number (0 if none)

+10	Word	Number of list entries (Ntot)

+12	Word	Number of visible list entries (Nvis)

+14	Word	First visible entry (0..Ntot-1)

+16	Word	Reserved - previous first visible

+18	Word	Current cursor position

+20	Word	Reserved - previous cursor position

+22	Byte	Dialog Font Number (1..12)

+23	Word	Height of each entry

+25	Wd24	24-bit address of string-list = list of 24-bit addresses

(strings may include tabs)

+28	Word	Tab position (in pixels from x)

+30	Wd24	24-bit address of on/off data table (Ntot bytes)

(1st byte = 0/255 = off/on for 1st entry, etc)

(whole data table must reside within one 16k block)

+33	Byte	Reserved (must be 0)

ob_vscroll:	(width must be 17)

+9		Byte	linked (list/listmulti) object number (0 if none)

+10	Word	Handle position (in pixels from top)

+12	Word	Handle size (in pixels)

ob_hscroll:	(height must be 17)

+9		Byte	Reserved (must be 0)

+10	Word	Handle position (in pixels from left)

+12	Word	Handle size (in pixels)

ob_spinner:

+9		Byte	Reserved

ob_dropbutton:(width and height must be 17)

+9		Byte	linked (ob_label) object number (0 if none)

+10-17	Words	x,y,w,h for droplist

+18	Word	Number of list entries (Ntot)

+20	Word	Number of visible list entries (Nvis)

+22	Word	Current cursor position

+24	Wd24	24-bit address of string-list = list of 24-bit addresses

Object definitions 2, 3, etc      As above

Terminator

+0		Byte	#00

out:	C .......	error in definition /  NC ..... OK

AF, BC, DE, HL, IX, IY corrupt

No.	Name

#	os_dialog_display	display current dialog	(only do ONCE for each os_dialog_define)

in:	none

out:	C .......	none to display!

NC .....	OK. Screen contents stored if required (see os_dialog_define)

AF, BC, DE, HL, IX, IY corrupt

#	os_dialog_remove	remove current dialog

in:	none

out:	C .......	none to remove!

NC .....	OK. Screen contents restored if required (see os_dialog_define)

AF, BC, DE, HL, IX, IY corrupt

#	os_object_define	(re)define one object in current dialog

in:	E = object number

CHL = 24-bit address of object definition (as for os_dialog_define)

N.B. Must not be longer than existing definition

out:	C .......	error -- object number out of range / error in object definition

NC .....	OK

AF, BC, DE, HL, IX, IY corrupt

#	os_object_get_define	get current definition of one object in current dialog

in:	E = object number

out:	C .......	error -- object number out of range

NC .....	CHL = 24-bit address of object definition (as for os_dialog_define)

AF, B, DE, IX, IY corrupt

#	os_object_display	display one object in current dialog

in:	E = object number

out:	C .......	error -- object number out of range

NC .....	OK

AF, BC, DE, HL, IX, IY corrupt

#	os_object_update	update display of one object in current dialog

in:	E = object number

out:	C .......	error -- object number out of range

NC .....	OK

AF, BC, DE, HL, IX, IY corrupt

No.	Name

#	os_set_focus		set focus object

in:	E = object number

out:	C .......	error -- no dialog / not focusable object / dialog has no focus

NC .....	OK

AF, BC, DE, HL, IX, IY corrupt

#	os_get_focus		get focus object

in:	none

out:	C .......	error -- no dialog / dialog has no focus

NC .....	E = focus object number

AF, BC, DE, HL, IX, IY corrupt

#	os_alert			do a simple 'alert' dialog (text & exitbuttons only)

in:	C,HL = 24-bit address of alert definition

DE = Total Data Length of definition

Byte 0...	ASCII. Each line 0-terminated, plus extra 0 to indicate end of text.

Byte		Number of buttons

Button 1	4 Words for keys ( 4 pairs of Ascii, Shift [#80=ignore] )

0-terminated ASCII text		( N.B. no repeat-flag byte! )

Button 2	ditto

etc

out:	C .......	error in definition

NC .....	A = Exit code (Number of Selected Button 0...n-1)

BC, DE, HL, IX, IY corrupt

#	os_report		display a simple 'alert'-style report (text only)

in:	C,HL = 24-bit address of report definition

DE = Total Data Length of definition

Byte 0...	ASCII. Each line 0-terminated

plus TWO extra 0s to indicate end of text

out:	NC/C ..	OK / error in definition

BC, DE, HL, IX, IY corrupt

n.b. this function stores the previous screen contents of the area covered by the report.

os_report_remove MUST be used to clear the report or the heap will misbehave.

#	os_report_remove	remove the previous report  (restore previous screen contents)

in:	none

out:	NC/C ..	OK / no report to remove

BC, DE, HL, IX, IY corrupt

THE TEXT EDITOR / CALCULATOR / DISPLAYER ...

No.	Name

#	os_text_edit		text edit/calculate/display function

in:	C,HL = 24-bit address of 32-byte parameter block:

+0	Byte	Mode   0=calc  1=display  2=edit displayed  3=display&edit  4=ed blank

+1	Byte	0=no highlight	1=do highlight				(Edit only)

+2	Word	Length of input text

+4	Wd24	24-bit address of input text

+7	Byte 	0=unlimited length single line

1...31=number of lines (128 char per line max).

+8	Words	X, Y, W, H of Window

+16	Word	Dialog Font No 1..12

+18	Word	Reserved (Font Size?)

+20	Byte	Reserved (Font Style?)

+21	Wd24	24-bit add of legal chrs list (Bank=0/1/2/3=all/num/filespec/dos-filespec)

+24	Byte	Length of  legal chrs list

+25	Wd24	24-bit address = destination for edited text		(Edit only)

+28	Word    Maximum bytes available at destination			(Edit only)

+30	Byte     0=disable scratchpad  1=enable scratchpad		(Edit only)

+31	Byte     0=no conversion  1=convert soft retns to hard in o/p	(Edit only)

out:	C .......	error	A = error code	0 ...	Input Length (too big)

1 ...	Font Does Not Exist

2 ...	Edit Window X/W (too narrow/wide / off-screen)

3 ...	Accent (no char / illegal char after accent code)

4 ...	Edit Window Y/H (too short/tall / off-screen)

5 ...	Input Text Generates Too Many Lines

6 ...	Output Length Error (max length = 0)

7 ...	Font Too Big

8 ...	Max Text Lines Out Of Range

9 ...	mode (1st byte of param block) out of range

NC .....	OK	A = exit code	bits 0..2	    000	Mouse Clicked Outside Edit Window

001 	Stop Key Pressed

010	TAB Key Pressed

011	Return / Enter Key Pressed

100	RED (Cancel) Key Pressed

101	GREEN (OK) Key Pressed

110	Cursor Up Key Pressed

111	Cursor Down Key Pressed

bit 3	    Shift-key held

bit 4	    Alt-Key held

bit 5	    Ctrl-Key held

bit 6	    Keypad key (0=Return, 1=Enter)

bit 7	    'Text altered' flag

BC = output length

All other regs corrupt

THE SPELL CHECKER

No.	Name

#	os_spell_check		spell checker function

in:	E =	0 = Check Word

C,HL = 24-bit address of 0-terminated word

must NOT be in range #802000-FFF

1 = Match Word

--- must only be used after Check Word has returned notfound

2 = Add Word to User Dictionary

--- must only be used after Check Word has returned notfound

3 = Delete Word from User Dictionary

C,HL = 24-bit address of 0-terminated word

must NOT be in range #802000-FFF

4 = List User Dictionary (Alphabetical Order)

C,HL = 24-bit address of buffer to list user dictionary in (max 11k)

must NOT be in range #802000-FFF

5 = List User Dictionary (Most-Recently-Used Order - MRU first...LRU last)

C,HL = 24-bit address of buffer to list user dictionary in (max 11k)

must NOT be in range #802000-FFF

6 = Get Spell-check Error Code

254 = Write User Dictionary to Flash

255 = Initialise (do not use - for OS only!!!!)

out:	0 check ........	A = 0...notfound / 1...found

1 match ........	A = no of words in list / #FF...error

CHL = address of list of 0-terminated words, 0 on end

2 add ............	A = #00...OK / #FF...error

3 delete..........	err ... ?

4/5 list ...........	HL = number of words in buffer

6 get error ......	A = HL = errorcode (see SPELL.Z80)

254 write ........	NC/C = OK/error, as filing function returns

THE DIALOG FONT SYSTEM ...

No.	Name

#	os_dialog_string			print string to screen in a dialog font

in:	CHL = 24-bit address of data ...

+0	Byte	Font No 1..12

+1	Byte	Alignment 0 / 1 / 2 = Left-Align / Centred / Right-Align

+2	Byte	Style (0 for now)

+3	Byte	Mode 0 / 1 / 2 / 3 = Opq / Wht Transp / Blk Transp / XOR

+4,+6	Words	x,y

+8...	ASCII	Text, 0-terminated

out:	C .......	Error in font/alignment/style/mode

NC .....	OK

AF corrupt, all other regs preserved

#	os_dialog_string_width		get pixel width of string in a dialog font

in:	CHL = 24-bit address of data ... (as for os_dialog_string, x/y irrelevant)

out:	C .......	Error in font/alignment/style/mode

NC .....	HL = pixel width of string

AF corrupt, all other regs preserved

THE TYPOGRAPHY FONT SYSTEM ...

No.	Name

#	os_typo_init		initialise typo font system (clear any existing slot contents)

in:	NONE

out:	NC/C ..	OK / not enough memory to initialise typofonts folder

AF corrupt, all other regs preserved

#	os_typo_getlist		get list of typo fonts available

in:	NONE

out:	B = number of entries in list

CHL = 24-bit address of list. 37 bytes for each entry ...

+0	   Word		16-bit ID

+2	   Byte		Styles		Bit0=Nrm 1=Bold 2=Italic 3=BoldItalic

+3	   Byte		RTF-family	0=unknown/default, 1=Times, 2=Swiss, etc

+4..+36	   33 bytes	ASCII Name	0-terminated and padded to 33 bytes

AF corrupt, all other regs preserved

#	os_typo_select		select typo font slot for def, clr, info & output functions

in:	E = slotno

out:	A = 0	OK

= 3	slotno out of range

= 4	OK, but no font in slot

#	os_typo_getselect	get selected typo font slot

in:	NONE

out:	E = current slotno

#	os_typo_def		install a typo font in the current slot

in:	HL = 16-bit ID

D = styles	Bit0=Nrm 1=Bold 2=Italic 3=BoldItalic   (Bit 0 MUST be 1. Usually D=#FF)

B = Point Size	6 / 8 / 10 / 11 / 12 / 14 / 18 / 24 / 36 / 72

out:	A = 0	OK

= 1	Font does not exist

= 2	Size out of range

= 7	not enough memory available in typofonts folder

= 8	slot already occupied

= 10	styles errror (bit 0 of D was not set)

F corrupt, all other regs preserved

#	os_typo_clr		clear the current typo slot

in:	none

out:	A = 0	OK

= 4	OK, but was no font in slot anyway

F corrupt, all other regs preserved

No.	Name

#	os_typo_getdef		get details of typo font installed in a slot

in:	E = slotno

out:	A = 0	OK .....	HL = 16-bit ID		(as os_typo_def)

D   = styles

B   = ptsz

= 3	slotno out of range

= 4	no font in slot

F corrupt, all other regs preserved

#	os_typo_info		get current typo font 'global' information (line pitch etc)

in:	NONE

out:	A = 0	HL = 16-bit address of	+0	Word	Line Pitch in 1/1800"

+2	Word	Max Ascender in 1/1800"

+4	Word	Max Descender in 1/1800"

+6	Word	Width of Widest Character in 1/1800"

= 4	no font in slot

#	os_typo_charinfo	get character info from current typo font slot

in:	C = Character number,  B = accent (0 if none)

D = style (only bit0=Bold, bits4/5=Super/Sub obeyed, bit6=Non-prop-numbers)

out:	A = 0	DE = Width in 1/1800"

HL = Height in 1/1800"

BC = Baseline Height in 1/1800"

= 4	no font in slot

= 5	Character or accent value(s) out of range (i.e. undefined in this font)

#	os_typo_charwidth	get character width from current typo font slot (faster than os_typo_charinfo)

in:	C = Character number,  B = accent (0 if none)

D = style (only bit0=Bold, bits4/5=Super/Sub obeyed, bit6=Non-prop-numbers)

out:	A = 0	DE = Width in 1/1800"

HL,BC corrupt

= 4	no font in slot

= 5	Character or accent value(s) out of range (i.e. undefined in this font)

#	os_typo_charscreen	put char to screen from current typo slot (obeys scr_clipwindow)

in:	C = Character number, B = accent (0 if none)

DE = x in PIXELS

HL = y in PIXELS

IX(lo) = style (bit0=Bold, bit1=Italic, bits4/5=Super/Sub, bit6=Non-prop-numbers)

IX(hi) = mode (0=Opaque / 1=White-Transp / 2=Black-Transp / 3=XOR)

out:	A = 0	OK
		   = 4	no font in slot

= 5	Character or accent value(s) out of range

= 6	Character 'cell' falls wholly outside Clipping Window

= 9	Character content falls wholly outside Clipping Window

#	os_select_character	select character (& optional accent) from on-screen dialog

in:	B = accent default, C = character default (0 in either if no default)

out:	NC .....	B = accent, C = character

C ........	User aborted

THE PRINT META-FILE FUNCTIONS ...

No.	Name

#	os_meta_open		start new meta-page

in:	CHL = 24-bit address of 13-byte parameter block ...

+0	Byte	Page orientation --- 0=portrait, non-0=Landscape

+1	Word	Page width in 1/1800"

+3	Word	Page height in 1/1800"

+5	Word	Page left-margin in 1/1800"

+7	Word	Page top-margin in 1/1800"

out:	A = 0	OK
		   = 7	not enough memory available to create meta-page

#	os_meta_font		set font of subsequent characters in meta-page

in:	HL = 16-bit font family ID

B = Point Size		6 / 8 / 10 / 11 / 12 / 14 / 18 / 24 / 36 / 72

out:	A = 0	OK
		   = 255	Metafile not open

#	os_meta_style		set style of subsequent characters in meta-page

in:	D = style (bit0=Bold, bit1=Italic & bits4/5=Super/Sub, bit6=Non-prop-numbers)

out:	A = 0	OK

= 255	Metafile not open

#	os_meta_y		set page y for baseline & top/bottom of subsequent characters

in:	HL = baseline y in 1/1800"

DE = top (min) y in 1/1800"

BC = bottom (max) y in 1/1800"

out:	A = 0	OK
		   = 255	Metafile not open

#	os_meta_char		put character to meta-page

in:	C = Character number

B = accent (0 if none)

DE = x in 1/1800"

out:	A = 0	OK
		   = 255	Metafile not open

No.	Name

#	os_meta_score		underscore character(s) on meta-page

in:	DE = left x in 1/1800"

BC = right x in 1/1800"

HL = y in 1/1800" (top of line)

IX  = line thickness in 1/1800"

out:	A = 0	OK
		   = 255	Metafile not open

#	os_meta_hline		horizontal line

in:	DE = left x in 1/1800"

BC = right x in 1/1800"

HL = y in 1/1800" (top of line)

IX  = line thickness in 1/1800"

out:	A = 0	OK
		   = 255	Metafile not open

#	os_meta_vline		vertical line

in:	DE = x in 1/1800" (left edge of line)

HL = top y in 1/1800"

BC = bottom y in 1/1800"

IX  = line thickness in 1/1800"

out:	A = 0	OK
		   = 255	Metafile not open

#	os_meta_print		send meta-page to printer

in:	B = 0 / 1 = full-resolution / half-resolution

out:	A = 0	OK
		   = 1	Not enough memory to print

= 2	Font not-found / unable-to-load error

= 3	Too many fonts error

= 4	User aborted print

= 5	Printer already in use

= 6	Printer type not valid

= 255	Metafile not open

#	os_meta_preview	send meta-page to screen preview

in:	B = 0 ...	auto-calculate screen view

B = 1 ...	CHL = 24-bit address of parameter block

+0	word	screen left x

+2	word	screen top y

+4	word	ratio (no of 1/1800" per screen pixel)

out:	A = 0	OK
		   = 1	Not enough memory

>		   = 7	Character off page

>		   = 8	Metafile data error (unrecognised record header)

= 255	Metafile not open

#	os_meta_close		close (de-initialise) meta-page

in:	none

out:	A = 0	OK
			   = 255	Metafile not open

THE HELP SYSTEM ...

No.	Name

#	os_set_help_context		set number of current help context

in:	HL = context number

out:	C .......	A=1 ... no help file found

A=2 ... context number is out of range

NC .....	OK, HL = previous context

#	os_release_help_context	release workspace used for help context

in:	NONE

out:	NONE

#	os_set_help_file			set filename for external help file

in:	CHL = 24-bit address of 0-terminated filename in flash System folder

out:	NC .....	OK

C .......	Could not find specified file in flash System folder

#	os_help_summary		display one-line help for current context

in:	NONE

out:	NONE

#	os_help_full			do full help dialog for current context

in:	NONE

out:	AF' corrupt

THE GRAPHICS FUNCTIONS ...

No.	Name

#	os_mouse_off		remove mouse from screen (if visible)

in/out:	NONE

#	os_mouse_on		replace mouse on screen (if was visible when off-ed)

in/out:	NONE

#	os_set_mouse_xy	set mouse x,y

in:	DE,HL = x,y

out:	C .......	x,y off screen

NC .....	OK

#	os_get_mouse_xy	get mouse xy

in:	NONE

out:	DE,HL = x,y

#	os_set_mouse_pointer	set mouse pointer

in:	E = pointer type       0 =	normal pointer

1 =	hourglass

2 =	I-beam

3 =	finger

4 =	palm

5 =	cross

out:	NONE

#	os_get_mouse_pointer	set mouse pointer

in:	NONE

out:	A = pointer type (as per os_set_mouse_pointer)

No.	Name

#	os_scr_direct		'direct' SCREEN graphic functions (unaffected by os_scr_setting)

in:	C, HL = 24-bit address of input parameters:

+0	Byte	direct graphic function number

+1...	Words	parameters

out:	A = 0	OK

= 1	Error in input parameter(s)

= 2

all functions executed with automatic mouse off/on

direct graphic function numbers & parameters (all co-ords in pixels relative to screen top-left):

Name		Meaning	  Parameters

dg_blk_white	White Block	   +1, 3, 5, 7	Words	x,y,w,h

dg_blk_black	Black Block	   +1, 3, 5, 7	Words	x,y,w,h

dg_blk_grey	Grey Block	   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	Grey Value 0..8=blk..wht

dg_blk_grey_xor  Grey Block (xor)   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	Grey Value 0..8=blk..wht

dg_blk_hilite	Hilite Block	   +1, 3, 5, 7	Words	x,y,w,h

dg_blk_move	Move Block	   +1, 3, 5, 7	Words	x,y,w,h

+9,11		Words	x2,y2

dg_blk_skewmove Skew Move 	   +1, 3, 5, 7	Words	x,y,w,h

+9,11		Words	x2,y2

+13		Byte	0/non-0 = LH edge flat/skewed

+14		Byte	0/non-0 = RH edge flat/skewed

+15		Word	Baseline height from y

dg_blk_skewwhite Skew White Blk  +1, 3, 5, 7	Words	x,y,w,h

+9..12	unused

+13		Byte	0/non-0 = LH edge flat/skewed

+14		Byte	0/non-0 = RH edge flat/skewed

+15		Word	Baseline height from y

dg_blk_skewlite	Skew Hilite 	   +1, 3, 5, 7	Words	x,y,w,h

+9..12	unused

+13		Byte	0/non-0 = LH edge flat/skewed

+14		Byte	0/non-0 = RH edge flat/skewed

+15		Word	Baseline height from y

dg_blk_copy	Copy Block	   +1, 3, 5, 7	Words	x,y,w,h

+9,11		Words	x2,y2

dg_blk_scroll	Scroll Block	   +1, 3, 5, 7	Words	x,y,w,h

+9,11		Words	dx,dy = pxls (pos=rt/dn, neg=lf/up)

+12		Byte	options: 0=move / non-0=copy

dg_multi_box	'Multi' Box	   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	fill colour (0-8=black-white  255=none)

+10		Byte	outline:	bits7-0 = w/b for up to 8 lines

+11		Byte	no of bits of outline to obey

+12		Byte	shadow depth (0-255!!!)

No.	Name

#	os_scr_direct  continued

The following functions are also available, but probably are not useful outside the OS itself ...

dg_alert_box	'Alert' Box	   +1, 3, 5, 7	Words	x,y,w,h

dg_dialog_box	'Dialog' Box	   +1, 3, 5, 7	Words	x,y,w,h

+9...		Ascii	Title Text, 0-terminated

dg_label	Text Label	   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	ignored (link)

+10		Byte	Dialog Font No 1...10

+11		Byte	Alignment (0/1/2 = L/C/R)

+12...		Ascii	Text, 0-terminated

dg_textbutton	Text Button	   +1, 3, 5, 7	Words	x,y,w,h

+9,11,13,15	Words	ignored (keycodes)

+17..		Ascii	Button Text, 0-terminated

dg_bmpbutton	Bitmap Button	   +1, 3, 5, 7	Words	x,y,w,h

+9,11,13,15	Words	ignored 	(keycodes in ob_ equivalent)

+17		Byte	ignored 	(repeat flag in ob_ equivalent)

+18,20,22,24	Words	dx,dy,w,h

+26		Wd24	Bitmap data address

If bank (+28) = 0, address (+26,7) = pre-defined

graphic no ....	0 = up-arrow	(w=17 h=17)

1 = down-arrow	(w=17 h=17)

2 = left-arrow	(w=17 h=17)

3 = right-arrow	(w=17 h=17)

>									4 = stop-sign	(w=39 h=39)

dg_dropbutton	Drop Button	   +1, 3, 5, 7	Words	x,y,w,h (w,h always 17,17)

dg_greybutton	Grey Big Button	   +1, 3, 5, 7	Words	x,y,w,h

dg_checkbox	Check Box	   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	#00 / #FF = unchecked / checked

dg_radio	Radio Button	   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	#00 / #FF = unchecked / checked

dg_textedit	Text Edit	   +1, 3, 5, 7	Words	x,y,w,h

+9...		32-Byte Params as ob_textedit in os_dia_define

dg_list		List Box		   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	ignored (link)

+10...		Bytes	Params as ob_list in os_dia_define

dg_vscroll	Vt Scroll Bar	   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	ignored (link)

+10...		Bytes	Params as ob_vscroll in os_dia_define

dg_hscroll	Hz Scroll Bar	   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	ignored (link)

+10...		Bytes	Params as ob_hscroll in os_dia_define

dg_spinner	Spinner		   +1, 3, 5, 7	Words	x,y,w,h

+9		Byte	ignored (reserved)

No.	Name

#	os_scr_window		set SCREEN clipping window

in:	CHL = 24-bit address of x,y,w,h (pixels relative to screen top-left)

out:	C .......	Requested window does not fit on screen

NC .....	OK

A corrupt, all other regs preserved

#	os_scr_getwindow	get SCREEN clipping window

in:	NONE

out:	CHL = 24-bit address of x,y,w,h (pixels relative to screen top-left)

C is always #80

A corrupt, all other regs preserved

THE FLOATING POINT FUNCTIONS ...

No.	Name

N.B.	FA (Float Accumulator) is a 6-byte buffer in bottom quarter. (Label FA defined in CONSTOS.Z80)

All float routines are register intensive so assume all registers are corrupt by these routines

#	os_float_function	perform floating-point mathematical function

in:	FA = 1st float

BCIXDE = 2nd float or DE=integer for os_float_float

HL = function number ...(constants defined in CONSTOS.Z80)

os_float_nop		no function

os_float_ten		FA = 10.0

os_float_two		FA = 2.0

os_float_one		FA = 1.0

os_float_zero		FA = 0.0

os_float_half		FA = 0.5

os_float_quarter		FA = 0.25

os_float_pi		FA = 3.1415927

os_float_half_pi		FA = 1.5707963

os_float_e		FA = 2.7182818

os_float_rand		FA = random 0.0...1.0

os_float_minus		FA = -FA

os_float_add_half		FA = FA + 0.5

os_float_mul10		FA = FA * 10

os_float_div10		FA = FA / 10

os_float_ceiling		FA = next int above FA

os_float_floor		FA = prev int below FA

os_float_sqrt		FA = square root(FA)

os_float_sin		FA = sin(FA in radians)

os_float_cos		FA = cos(FA in radians)

os_float_tan		FA = tan(FA in radians)

os_float_sinh		FA = hyperbolic sin(FA)

os_float_cosh		FA = hyperbolic cos(FA)

os_float_tanh		FA = hyperbolic tan(FA)

os_float_log10		FA = log(FA) base 10

os_float_log		FA = log(FA) base e

os_float_atan		FA = arc tan(FA) in radians

os_float_exp		FA = e to power FA

os_float_add		FA = BCIXDE + FA

os_float_sub		FA = BCIXDE - FA

os_float_mul		FA = BCIXDE * FA

os_float_div		FA = BCIXDE / FA

os_float_mod		FA = BCIXDE mod FA (untested)

os_float_pow		FA = FA ^ BCIXDE     (I think!)

os_float_compare		Z80 flags set as result of test FA == BCIXDE

os_float_min		FA = min(FA, BCIXDE)

os_float_max		FA = max(FA, BCIXDE)

os_float_cmp_ge		HL = 0 (false)/1 (true) for BCIXDE >= FA

os_float_cmp_gt		HL = 0 (false)/1 (true) for BCIXDE > FA

os_float_cmp_le		HL = 0 (false)/1 (true) for BCIXDE <= FA

os_float_cmp_lt		HL = 0 (false)/1 (true) for BCIXDE < FA

os_float_cmp_eq		HL = 0 (false)/1 (true) for BCIXDE == FA

os_float_cmp_ne		HL = 0 (false)/1 (true) for BCIXDE <> FA

os_float_float		convert integer DE to float in FA

os_float_int		convert FA to an integer in HL

os_float_sgn		set HL = -1, 0 or 1 depending on FA

os_float_rand_seed		set seed for os_float_rand from FA

out:	C .......	A = Error Code	(defined in CONSTOS.Z80)

1 = e_overflow

2 = e_div_zero

3 = e_unknown_function

4 = e_illegal_sqrt

5 = e_too_long	(float_to_ascii only)

NC .....	FA, HL or Flags = float result depending on function

No.	Name

#	os_float_to_ascii	convert float to ASCII string

in:	FA = float

CHL = 24-bit address of o/p buffer (must be at least 70 bytes)

D = ASCII format options

bit 2	negative "-nnnn" if 0, or "(nnnn)" if 1

bit 3	thousands separators inserted if = 1

bit 4	decimal point is "." if 0 or "," if 1

bit 5	unused if bit 3 = 0, else 0 => use commas, 1 => use spaces

E = number of digits to display in two nybbles...

E:0..E:3	  digits after decimal point (0..7). #F is special value as described below

E:4..E:7  significant figures before decimal point (0..7), #F is special value

E = #FF (both E:0..E:3 and E:4..E:7 = #F) convert number using best possible

method. If FA>0.0001 and FA < 10,000,000 then use normal form but

outside this range use exponential format.

E = #Fx display exponential with x significant figures

out:	Z......	A = 0 if successful

HL = address of NULL at end of the string

NZ	A = e_too_long 		Number will not fit in buffer

#	os_ascii_to_float	convert ASCII string to float

normal mmm.nnn or exponential form mmm.nnnEpp

in:	CHL = 24-bit address of ASCII (terminated by illegal character e.g. 0)

out:	Z .....	A=0, FA = float

NZ ......	A=e_overflow - impossible number

THE CLIPBOARD (SCRATCHPAD) ...

No.	Name

#	os_scratch_clear	clear any existing data from scratchpad

in:	none

out:	none

#	os_scratch_append	append data to scratchpad

in:	CHL = address of data

DE = data length

out:	A = 0	OK

= 1	No memory free to create scratchpad

= 2	Too much data (scratchpad would exceed 16k limit)

#	os_scratch_get		get data from scratchpad

in:	NONE

out:	C ........ 	A=255	No data

NC .....	A=0	C,HL = 24-bit address of data

DE = data length

IX = appID of data owner

THE FILING FUNCTIONS ...

For further details of these functions, refer to the document Low-Level Disk Interface for the Amstrad PCW 'Anne' (Ranger Computers Ltd), but note the following ...

N.B.		Except for os_set_device and os_get_device, the os_xxxx_xxxx Filing Functions are linked directly to Ranger Software's r_xxxx_xxxx functions with one important difference in input parameters:

In the Ranger documentation the A register is shown as denoting the device (1=Floppy or 2=Flash) for most r_xxxx_xxxx functions, but in Rosanne os_set_device is used to set the current device before performing the equivalent os_xxxx_xxxx function. For any r_xxxx_xxxx function which expects A=device, the OS loads the A register with the current device setting before passing control from the os_xxxx_xxx code to the r_xxxx_xxxx code. (In calling Rosanne os_xxxx_xxxx functions, the A register is used to denote which os_xxxx_xxxx function is being called ! ).

e.g. os_disk_space ... to get the free space on the Floppy drive, do ...

ld	b,1

oscall	os_set_device

oscall	os_disk_space

... even though r_disk_space is shown in the Ranger doc as using A to denote the device.

The current device setting affects all (& only) the functions listed in the Ranger documentation as having A as an input parameter denoting device=Flash/Floppy. The current setting is not corrupted by any other functions, so once set to e.g. Flash it does not have to be set to Flash again unless it has been set to Floppy.

All relevant Filing Functions return standard error codes as follows:

13      	WRITE_PROTECT		the disk write protect tab is open

15	NOT_READY			incorrect response from drive hardware

22	WRONG_DISK			disk has been changed

52      	CANNOT_MAKE		no room on disk

A0	NO_DISK			no disk in drive

A3      	DISK_FULL			no room left on disk

B9	NOT SUPPORTED		device = Flash / disk=CPM

C4	DIR_TABLE_FULL		root directory table full

C9	NOT_ANNE			no ANNE structure, but given an ANNE filename

D1 - DB					floppy drive or flash hardware error

Additional error codes for each function are listed below

All registers which are not specified on exit are assumed corrupted (except AF', BC', DE', HL' which are always preserved)

FILEINFO structure

Many functions set up a FILEINFO structure ... this is as listed in the Ranger documentation. This structure can be found by using os_get_fileinfo which returns the 24-bit address of FILEINFO.

FLASH FILE HEADER structure

Files in the Flash Drive contain a header as specified in the Ranger documentation. When data is read from a Flash file using os_read_file or os_copy_file, it does not include the header. And pointer value 000000 addresses the first byte after the header.

> NOTE: os_app_exit automatically closes any opened read and/or write files.

No.	Name

#	os_dos_init		intialise DOS (incl Log Floppy if present and Check Flash)

in:	none

out:	NC .....	OK

C ........	Error code in A	#08	Not Enough Memory to initialise DOS

#CA	Flash Drive requires purging

#	os_set_device		set current device for DOS functions - NO r_xxxx equivalent

in:	B .......	1 = Floppy /  2 = Flash

out:	NC .....	OK

C ........	Invalid device number

#	os_get_device		get current device for DOS functions - NO r_xxxx equivalent

in:	none

out:	B .......	1 = Floppy /  2 = Flash

No.	Name

#	os_park_heads		park heads of current device

in:	none

out:	none

#	os_change_disk		log new disk (after WRONG_DISK error) in current device

in:	none

out:	NC / C = OK / Error	(No effect if current device is Flash)

#	os_check_disk		determine whether disk has changed since last operation

in:	none

out:	NC .....	OK		Always NC if current device is Flash

C .......	A = #22		Disk has been changed

#	os_disk_space		get amount of free 512-byte blocks on selected device

in:	none

out:	NC .....	OK	CHL = number of 'free' 512-byte blocks

(for Flash this includes those pending erasure)

DE = numbers of 512-byte blocks pending erasure (Flash only)

C ........	Error

N.B. Each 512-byte block of Flash Drive represents 510 bytes of storage space

#	os_flash_percent	get percent free of non-system flash

in:	B = 0 for percentages, non-0 for cabinet sizing

out:	B=0 	A = percent free now

B = percent pending erasure

C = percent which would be free if all pending erased (i.e. A+B)

B=1 	HL = total cabinet size in blocks

DE = system folder occupancy in blocks

AF,BC corrupt

#	os_get_cwd		get current working directory (DOS-compatible floppy only)

in:	HL = 16-bit address of buffer for output in bottom quarter of memory.

Must be at least 65 bytes long.

out:	NC .....	OK	HL still points to buffer with 0-terminated path

C ........	Error

#	os_set_cwd		set current working directory (DOS-compatible floppy only)

in:	HL = 16-bit address of 0-terminated path in bottom quarter of memory

Must be enough space at HL=> to accept new path

out:	NC .....	OK	HL still points to buffer with 0-terminated new path

C ........	Error	HL still points to buffer with original input path

A = #03	  Path Not Found

#B7	  Invalid name - path contains invalid chars

#BA  Too Big - new path exceeds 64 chars

No.	Name

#	os_get_flashfolder	get current flash folder - NO r_xxxx equivalent

in:	none

out:	NC .....	OK	B = current folder number 0..16 / #FF for wildcard

#	os_set_flashfolder	set current flash folder - NO r_xxxx equivalent

in:	B = 0..16 / #FF for wildcard

out:	NC .....	OK

C ........	Error	A = #03  Invalid folder number

#	os_get_cpmfolder	get current CP/M-disk folder (user group) - NO r_xxxx equivalent

in:	none

out:	NC .....	OK	B = current user number 0..15 / #FF for wildcard

C ........	Error	A = #03  Folder not found (Invalid folder number)

#CB  Not CP/M Disk

#	os_set_cpmfolder	set current CP/M-disk folder (user group) - NO r_xxxx equivalent

in:	B = 0..15 / #FF for wildcard

out:	NC .....	OK

C ........	Error	A = #CB  Not CP/M Disk

#	os_find_first		find first file or directory in current directory, current device

in:	HL = 16-bit address of 0-terminated filespec in bottom quarter of memory

DE = 16-bit address of 0-terminated extension (incl .) in bottom quarter of memory

0000 if no extension specified

B = search type	#00 = normal (search Catalog file / DOS root only)

#01 = extended (return attribute, size & time/date info)

#10 = directory (return directory names only)

out:	NC .....	OK	FILEINFO structure filled with file / directory info

C ........	Error	A = #02		File Not Found (no match)

#	os_find_next		find next file or directory in current directory, current device

in:	none

out:	NC .....	OK	FILEINFO structure filled with file / directory info

C ........	Error	A = #01	  Invalid (no os_find_first done yet!)

#12	  No more files found

#	os_get_fileinfo		get FILEINFO address

in:	none

out:	CHL = 24-bit address of FILEINFO record (see Ranger documentation)

No.	Name

#	os_set_filedate		set date&time stamp of current file open for write access

in:	HL = time (DOS format)

DE = date (DOS format)

out:	NC .....	OK

C ........	Error	A = #02 File Not Open

#	os_open_file	open file for read or write

in:	HL = 16-bit address of 0-terminated filename in bottom quarter of memory

DE = 16-bit address of 0-terminated extension (incl .) in bottom quarter of memory

0000 if no extension specified

B = access mode ... 0 / 1 = read / write

out:	NC .....	OK

C ........	Error	A = #02	  File Not Found

#04	  Too many open files for requested access mode

#05	  Access Denied (r/o when opened for write)

#	os_close_file	close file for read or write

in:	B = access mode ... 0 / 1 = read / write

out:	NC .....	OK

C ........	Error	A = #02	  File Not Found (i.e. No Open File)

#	os_write_file	write sequential data to file opened for output

data is always written to the current file position

in:	CHL = 24-bit address to write data from

BDE = 24-bit length to write (if will run over a 16k block end, the RAM must be 'owned')

out:	NC .....	OK

C ........	Error	A = #02	  File Not Open

#08	  Insufficient RAM to write from (get_memlist error)

#A3	  Disk Full (no data has been written)

#	os_read_file	read sequential data from file opened for input

data is always read from the current file position

in:	CHL =	24-bit address to load data to (there must be enough space at CHL to allow 			RAM to be filled up with the remainder of the last 512-/510-byte block read).

BDE =	24-bit length to load (if it will over a 16k block end, the RAM must be 'owned')

out:	NC .....	OK

C ........	Error	A = #02	  File Not Open

#08	  Insufficient RAM to read to (get_memlist error)

#A1	  Premature EoF (BDE = amount NOT loaded, CHL = filesize)

#	os_copy_file	copy data from file opened for input to file opened for output

data is always read from and written to the file start positions

in:	BDE =	24-bit length to copy

out:	NC .....	OK

C ........	Error	A = #02	  File Not Open (read / write / both)

#08	  Not enough memory available from OS

#A1	  Premature EoF (BDE = amount NOT loaded, CHL = filesize)

#A3	  Disk Full (no data has been written)

No.	Name

#	os_set_attrib	set file / directory attributes

in:	HL = 16-bit address of 0-terminated filename in bottom quarter of memory

DE = 16-bit address of 0-terminated extension (incl .) in bottom quarter of memory

0000 if no extension specified

B = attributes 	Bit 0 = Read Only,  Bit 1 = Hidden,  Bit 2 = System,  Bit 5 = Archive

out:	NC .....	OK	FILEINFO filled with relevant info

C ........	Error	A = #02	  File Not Found

#05	  Access Denied (r/o file)

#	os_create_directory	create new directory within current working directory, current device

in:	HL = 16-bit address of 0-terminated directory name in bottom quarter of memory

out:	NC .....	OK

C ........	Error	A = #13	  Write-Protected

#50	  File/Directory name already exists

#	os_structure		create ANNE folder struture and CATALOG.TXT file on current device

in:	none

out:	NC .....	OK

C ........	Error	A = #05	  Access Denied (disk is not blank)

#	os_remove_directory	remove directory within current working directory, current device

in:	HL = 16-bit address of 0-terminated directory name in bottom quarter of memory

out:	NC .....	OK

C ........	Error	A = #02	  Directory Not Found

#05	  Access Denied (r/o directory or files exist within it)

#B7	  Invalid name (wildcards used in directory name)

#	os_delete_file		delete file in current directory, current device

in:	HL = 16-bit address of 0-terminated filename in bottom quarter of memory

DE = 16-bit address of 0-terminated extension in bottom quarter of memory

0000 if no extension specified

out:	NC .....	OK	FILEINFO filled with relevant info

C ........	Error	A = #02	  File Not Found

#05	  Access Denied (r/o file)

#	os_rename_file		rename file / directory in current directory, current device

in:	HL = 16-bit address of 0-terminated existing filename in bottom quarter of memory

DE = 16-bit address of 0-terminated existing extension in bottom quarter of memory

IX = 16-bit address of 0-terminated new filename in bottom quarter of memory

IY = 16-bit address of 0-terminated new extension in bottom quarter of memory

B = #00 ... Rename FILE / #10 ... Rename DIRECTORY

out:	NC .....	OK	FILEINFO filled with relevant info for new name

C ........	Error	A = #02	  File Not Found

#05	  Access Denied (r/o file)

#50	  File/Directory name already exists

No.	Name

#	os_get_ptr		get current position of file pointer for read file

in:	none

out:	NC .....	OK	BDE = pointer position

C ........	Error	A = #C8  No file open for read access

#	os_set_ptr		set current position of file pointer for read file (Flash only)

in:	BDE = new value for pointer

out:	NC .....	OK

C ........	Error	A = #A1  EOF (Requested position is off end of file)

#C8  No file open for read access

#	os_format_flash		format and set up the Flash Drive (all existing data is lost)

in:	none

out:	NC .....	OK

C ........	Error	EXTREMELY SERIOUS!!!

Flash hardware error codes only reported

#	os_purge_flash		purge flash inconsistencies

in:	none

out:	NC .....	OK

C ........	Error	EXTREMELY SERIOUS!!!

Flash hardware error codes only reported

#	os_set_file_defaults	set filing system default settings

in:	B = sort order 0/1/2 = alpha/date/size

C = default storage device 1/2 =floppy/flash

out:	NC ......	OK

C .........	out of range

#	os_get_file_defaults	get filing system default settings

in:	none

out:	B = sort order 0/1/2 = alpha/date/size

C = default storage device 1/2 =floppy/flash

No.	Name

#	os_file_selector		file selector

in:	CHL = 24 bit address of a 21 byte parameter block:

+0	byte	Mode	0 = Load    1 = Save    2 = Import    3 = Export
The following may be added to the basic Mode:
    #80 = Allow change of device (ignored in alien modes)
    #40 = Allow change of folder
    #20 = Suppress "flash is nearly full" warning
    #10 = Allow overwrite of existing files (after a confirmation alert)
    #08 = Allow access to system folder

+1	wd24	Title	Address of Title, 0-terminated, e.g. "EDIT DOCUMENT"

+4	wd24	Prompt	Address of Prompt, 0-term e.g. "Select document then press OK"

+7	byte	Init_Device	Initial device, as os_set_device, ignored in alien modes.

+8	wd24	Init_Folder	Address of Initial folder name, 0-terminated, or #00,XXXX for first legal folder or #00,XXNN to select folders by number (cabinet only).  Ignored for Import and Export.

+11	wd24	Init_Name	Address of Initial filename (Save/Export only) or #00,XXXX if none.

+14	3 bytes	Extension	Space-pad if less than 3 characters, #00 in first byte if none.  For For LOAD, extension to restrict search to.
For SAVE, extension to put onto saved file.
For EXPORT, default DOS suffix.  For IMPORT, ignored.

+17	byte	Type_mask	bit 2 = Allow MS-DOS 1.44M discs
bit 3 = Allow MS-DOS 720K discs
bit 4 = Allow CP/M 720K discs (Import only)
Ignored for Load and Save (i.e. native modes).

+18	wd24	Type_alert	The text of an alert to show for an invalid disc type
One or more 0-terminated strings, followed by another 0 byte
#00,XXXX means use the standard FS type-error alert.

out:	NC ......	OK

A bits 5-0 = media type (1 bit only set)	bit 0 = PcW16 1.44Mb

bit 1 = PcW16 720k

bit 2 = MS-DOS 1.44Mb

bit 3 = MS-DOS 720k

bit 4 = CP/M 720k

bit 5 = Flash

A bit 7 = overwrite flag = 1 if file already exists (overwrite has been OK-ed by user)

HL = 16-bit address of 0-terminated filename selected (in block #80 in bottom qtr)

- 32 chars max if Load & Save / DOS-legal for Import & Export

DE = 16-bit address of 0-terminated extension (in block #80 in bottom qtr) / #0000 if none

C ........	Error

A = code ...	1 = Aborted

2 = FS already on-screen

3 = Memory Folder error

4 = Memory error (couldn't get required 32k RAM to operate)

5 = Init_Folder does not exist, and 'Change Folder' not allowed

6 = Init_Device does not exist

7 = Flash error

8 = Silly parameters (e.g. attempt to Export to CP/M)

DE, HL corrupt

No.	Name

#	os_file_selector continued

N.B.	The file-selector dialog remains on-screen when os_file_selector function returns - use os_file_selector_off  to remove it again.

But note the filename & extension pointed to by HL & DE are no longer valid once you have, so the usual sequence for opening a user-selected file is:

os_file_selector

os_file_open

os_file_selector_off.

Additionally, while the dialog is still on-screen you must NOT use os_report / os_dialog_display unless you use a matching os_report_remove / os_dialog_remove before os_file_selector_off.

#	os_file_selector_off	remove file selector from screen

in:	none

out:	NC .....	OK - Data pointed to by DE & HL from os_file_selector invalidated.

C .......	FS wasn't on screen

THE PRINT FUNCTIONS ...

No.	Name

#	os_get_printer_dpi	get printer dpi

in:	NONE

out:	A = 0=200dpi / 1=300dpi / 2=360dpi / 3=reserved

#	os_print_startjob	initialise print system

in:	NONE

out:	NC .....	OK

C .....	A = 2	User aborted

= 4	Printer already in use

= 5	Printer type not yet registered by Setup

#	os_print_endjob		'de-initialise' print system (!)

in:	NONE

out:	NONE

#	os_print_stripe		print current stripe

in:	B = 0 ..	print at full res	(stripe_dpi)

B = 1 ..	print at lo-res	(stripe_dpi/2)

HL = advance linefeed amount in lines @ stripedpi

out:	NC ..... 	OK

C .......	A = 1	Memory problem (i.e. try again at lo-res if not already)

= 2	User aborted

= 3	System aborted / Print job not started / stripe_folder incomplete

= 5	Printer type not valid

= 6	stripe_folder incomplete

#	os_print_screen		print screen

in:	none

out:	NC .....	OK

C .......	A = 1	Memory problem (i.e. free up mem if poss & try again)

= 2	User aborted

= 3	Obscure internal error!

= 4	Printer already in use

= 5	Printer type not valid

= 6	Obscure internal error!

BIOS functions

Each BIOS function is accessed by calling bios_jumpblock + 3*function_no.

(bios_jumpblock defined in CONSTOS.Z80)

So ... to call BIOS function #07, do CALL bios_jumpblock + 21

No.	Name

#00	bios_restart	no function yet!

#01	bios_getkey	get key from key buffer (if any)

in:	NONE

out:	Z if no key

NZ if is one, A=ascii, C=shiftstate: bits 4,2,1,0 = Kypd, Ctrl, Alt, Shift

other flags corrupt

N.B. bios_getkey does not work if events are switched on (which they are by default, and currently can't be turned off) i.e. bios_getkey doesn't work !  Using bios_putkey will generate an OS key event instead of a readable entry in the bios keyboard buffer.

#02	bios_putkey	put key into key buffer

in:	A=ascii, C=shiftstate: bits 2,1,0 = Ctrl, Alt, Shift

out:	none

AF corrupt

#03	bios_keystate	get up/down status of a key

in:	A=keyno

out:	A = 0 / non-0 = key up / down

F corrupt

#04	bios_setleds	set state of keyboard leds

in:	A = leds (bit0=spare, bit1=capslock, bit2=appointments)

out:	F corrupt

#05	bios_getleds	get current state of keyboard leds

in:	none

out:	A = as bios_setleds

#06	bios_setrate	set keyboard typematic rate/delay

in:	A = rate/delay byte (ask SG or IBM!!!!)

out:	F corrupt

#07	bios_mousoff	turn mouse screen cursor (pointer) off

in:	none

out:	all flags & registers preserved

A variable mous_vis is maintained in the OS which records the level of 'hiddenness' of the Mouse cursor (pointer). bios_mousoff decrements this variable, and removes the cursor (pointer) from the screen if mous_vis was exactly 1 (and becomes 0). mous_vis = 1 when AP first launched.

#08		bios_mouson	turn mouse screen cursor (pointer) on

in:	none

out:	all flags & registers preserved

bios_mouson increments mous_vis, and places the cursor (pointer) on the screen if mous_vis was exactly 0 (and becomes 1). mous_vis = 1 when AP first launched.

#09	bios_getmsxy	get mouse x,y

in:	none

out:	DE=mousex (0...639), HL=mousey (0..479)

F corrupt

#0A	bios_setmsxy	set mouse x,y

in:	DE=mousex (0..639), HL=mousey (0..479)

out:	DE,HL=corrected mousex,y (if outside range)

F corrupt

#0B	bios_getmsbt	get mouse button state

in:	none

out:	Z if none pressed, NZ if any of L/C/R pressed

A=button state   bits 2=L, 1=C, 0=L   0=unpressed/1=pressed

other flags corrupt

#0C	bios_msupdate update mouse screen cursor (pointer) position

in:	none

out:	AF, BC, DE, HL corrupt

Should not normally be necessary. Only moves the pointer if it needs to.

#0D	bios_setmsptr	set mouse cursor (pointer) style

in:	A=pointer number 0...15

out:	all regs preserved

#0E	bios_getmsptr	get mouse cursor (pointer) style

in:	none

out:	A=pointer number 0..15.

#0F	bios_defmsptr	define mouse cursor (pointer) style for pointer number 15

in:	HL=16-bit address of 68 bytes:	   0..3  = words for xoffset,yoffset of hot-spot

4..35 = 2*16 byte cursor design

36..67 = 2*16 byte mask design

Cursorbit   Maskbit   Screen result             Cursorbit   Maskbit   Screen result

0                  0                 0                                   0               1         unchanged

1                  0                 1                                   1               1         inverted

out:	AF, BC, DE, HL corrupt

#10	bios_gettime	get system time

in:	none

out:	E   = ticks	0..255

D   = seconds	0..59

C   = minutes	0..59

B   = hours	0..23

F corrupt

#11	bios_settime	set system time

in:	D   = seconds	0..59

C   = minutes	0..59

B   = hours	0..23

out:	F corrupt, A B C corrected if out of range

#12	bios_getdate	get system date

in:	none

out:	A   = day	0..31

C   = month	0..11

B   = year	0..127 where 0 means 1980

F corrupt

#13	bios_setdate	set system date

in:	A   = day	0..31

C   = month	0..11

B   = year	0..127 where 0 means 1980

out:	F corrupt, A B C corrected if out of range

#14	bios_setmem0	set memory block for #0000...#3FFF			- don't use this !!!!!!!!

in:	A = block number (#00..#7F=ROM, #80..#FF=RAM)

out:	all flags & regs preserved

#15	bios_setmem1	set memory block for #4000...#7FFF

in/out:	as bios_setmem0

#16	bios_setmem2	set memory block for #8000...#BFFF

in/out:	as bios_setmem0

#17	bios_setmem3	set memory block for #C000...#FFFF

in/out:	as bios_setmem0

#18	bios_getmem0	get memory block for #0000...#3FFF

in:	none

out:	A = block number (#00..#7F=ROM, #80..#FF=RAM)

all flags & other regs preserved

#19	bios_getmem1	get memory block for #4000...#7FFF

in/out:	as bios_getmem0

#1A	bios_getmem2	get memory block for #8000...#BFFF

in/out:	as bios_getmem0

#1B	bios_getmem3	get memory block for #C000...#FFFF

in/out:	as bios_getmem0

#1C	bios_di		disable interrupts

in:	none

out:	all flags & regs preserved

#1D	bios_ei		enable interrupts

in:	none

out:	all flags & regs preserved

#1E	bios_gi		get interrupt-enable status

in:	none

out:	Z and A=0 if interrupts disabled

NZ and A=255 if interrupts enabled

#1F	bios_clrscrn	clear screen to black / white

in:	A = #00 (black)  #FF (white)  #AA (50% grey)

out:	AF, C corrupt. All other regs preserved

#20	bios_setink	set foreground ink to black / white

in:	A = 0 (black) or 1 (white)

out:	F, C corrupt. All other regs preserved

#21	bios_getink	get foreground ink

in:	none

out:	A = 0 (black) or 1 (white)

All other regs preserved

#22	bios_setmode	set graphics mode

in:	A = 0 (Opaque)  1 (White=Transparent)  2 (Black=Transparent)  3 (XOR)

out:	F, C corrupt. All other regs preserved

#23	bios_getmode	get graphics mode

in:	none

out:	A = 0 (Opaque)  1 (White=Transparent)  2 (Black=Transparent)  3 (XOR)

All other regs preserved

#24	bios_setpatt	set 8*8-bit pattern for bios_block

in:	HL = 16-bit address of 8 bytes of pattern data

out:	AF, BC, HL corrupt. All other regs preserved

#25	bios_getpatt	get 8*8-bit pattern

in:	none

out:	HL = 16-bit address of (copy of) 8 bytes of pattern data

All other regs preserved

#26	bios_pixel	plot a pixel in foreground ink using graphics mode

in:	DE = x   HL = y

out:	AF, C corrupt. All other regs preserved

#27	bios_getpxl	get state of a pixel

in:	DE = x,   HL = y

out:	A = 0 (black) or 1 (white)

F, HL corrupt. All other regs preserved

#28	bios_line		draw line in foreground ink using graphics mode

in:	HL = 16-bit address of 4 words: x1, y1, x2, y2

out:	AF, BC, HL corrupt. All other regs preserved.

#29	bios_rectang	draw rectangle in foregrnd ink using graphics mode

in:	HL = 16-bit address of 4 words: x1, y1, w, h

out:	AF, BC, HL corrupt. All other regs preserved.

#2A	bios_block	draw filled (rectangular) block in foerground ink using graphics mode

in:	HL = 16-bit address of 4 words x1, y1, w, h

out:	AF, BC, HL corrupt. All other regs preserved.

#2B	bios_patblock	draw pattern-filled (rectangular) block in graphic pattern using graphics mode

in:	HL = 16-bit address of 4 words x1, y1, w, h

out:	AF, BC, HL corrupt. All other regs preserved.

#2C	bios_storebitmap	store rectangular screen bitmap in memory

in:	A = bitmap no 1..15	(0 reserved for OS)

HL = 16-bit address of 4 words: x, y, w, h (pxls)

out:	AF, BC, DE, HL corrupt. All other regs preserved

#2D	bios_restorebitmap	restore rectangular screen bitmap from memory

in:	A = bitmap no 1..15	(0 reserved for OS)

DE = x, HL = y

out:	AF, BC, DE, HL corrupt. All other regs preserved

#2E	bios_plotbitmap		plot rectangular screen bitmap

in:	HL = 16-bit address of bitmap data	+0	x coord	(word)

+2	y coord	(word)

+4	width in pxls	(word)

+6	height in pxls	(word)

+8	24-bit address of data

out:	AF, BC, DE, HL corrupt. All other regs preserved

#2F	bios_plotmaskedbitmap	plot non-rectangular screen bitmap

in:	HL = 16-bit address of bitmap data	+0	x coord	(word)

+2	y coord	(word)

+4	width in pxls	(word)

+6	height in pxls	(word)

+8	24-bit address of data

+11	24-bit address of mask data (0=transp)

out:	AF, BC, DE, HL corrupt. All other regs preserved

#30	bios_plotmaskvari	plot vari-sized non-rectangular screen bitmap

in:	HL = 16-bit address of bitmap data as bios_plotmaskedbitmap

DE = element size 01 / 02 / 03

out:	AF, BC, DE, HL corrupt. All other regs preserved

#31	bios_getbitmap	get rectangular screen bitmap to memory

in:	HL = 16-bit address of 4 words: x, y, w, h (pxls)

DE = 16-bit address of buffer to put ((width+7) DIV 8)*height bytes of data from screen

out:	AF, BC, DE, HL corrupt. All other regs preserved

#32	bios_bitmapinfo	get address of stored bitmap pointer info

in:	none

out:	HL = 16-bit address of bmadtab (7 bytes per bitmap = addr24,w,h)

All other regs preserved

#33	bios_setfont	set font for bios_string

in:	A = font number 0..15

out:	F, C corrupt. All other regs preserved.

#34	bios_getfont	get font

in:	none

out:	A = font number 0..15

All other regs preserved

#35	bios_setalign	set text alignment for bios_string

in:	A = 0 (Left)   1 (Centre)   2 (Right)

out:	F, C corrupt. All other regs preserved.

#36	bios_getalign	get text alignment

in:	none

out:	A = 0 (Left)   1 (Centre)   2 (Right)

All other regs preserved

#37	bios_string	print text in current font using text alignment & graphics mode

in:	HL = 16-bit address of string data:	+0	x coord	(word)

+2	y coord	(word)

+4	text length 	(byte)

+5...	text...

out:	AF corrupt. All other regs preserved

#38	bios_putchar	print 1 character in current font using graphics mode

in:	DE = x coord

HL = y coord

C = accent (0 if none)

A = character

out:	AF, BC, DE, HL corrupt. All other regs preserved

#39	bios_clipstring	print text in current font etc., clipped to width in DE

in:	HL = 16-bit address of string data:	+0	x coord	(word)

+2	y coord	(word)

+4	text length 	(byte)

+5...	text...

DE = width to restrict printing to

out:	AF corrupt. All other regs preserved

#3A	bios_diskio	bios disk functions ...

in:	A = function no	1 = dRead

2 = dWrite

3 = dLogDisk

4 = dReset

5 = dFormat

6 = dShutDown

B = device 	1 = Floppy

2 = Flash

3 = Hard Drive

Function	Other Regs

dRead		see DOS.Z80 for details !!!

dWrite		see DOS.Z80 for details !!!

dLogDisk	see DOS.Z80 for details !!!

dReset		see DOS.Z80 for details !!!

dFormat	see DOS.Z80 for details !!!

dShutDown	see DOS.Z80 for details !!!

#3B	bios_intserve		install a user-defined interrupt server

in:	CHL = 	24-bit far execution address (top 2 bits of HL ARE significant as per farcall)

This must NOT be in range #802000-#802FFF

Code at CHL will be executed on every interrupt, with A=interrupt status register contents

Must return with ALL registers intact, and must NOT enable interrupts.

out:	AF corrupt

#3C	bios_intkill		remove user-defined interrupt server

in:	none

out:	AF corrupt

#3D	bios_printbyte		send a single byte to printer	(NOT VALID WHEN PRINTER = OKI 400w)

in:	A = byte to print

out:	NC = OK (A unchanged) / C = aborted (A=2)

#3E	bios_printstring		send a string to printer		(NOT VALID WHEN PRINTER = OKI 400w)

in:	HL = address of string, 1st byte = length (max 255)  N.B. HL must NOT be in range #4000-7FFF

out:	NC = OK, C = Aborted.  HL corrupt

Keyboard Codes

Key		       Normal	 Shift	         Alt	   Shift-Alt	Ctrl	      Shift-Ctrl

| STOP (Esc) | #FE | #FE | #FE | #FE | #FE | #FE |
| --- | --- | --- | --- | --- | --- | --- |
| f1 | #F1 | #F1 |  | #F1 | #F1 | #F1 |
| f2 | #F2 | #F2 |  | #F2 | #F2 | #F2 |
| f3 | #F3 | #F3 |  | #F3 | #F3 | #F3 |
| f4 | #F4 | #F4 |  | #F4 | #F4 | #F4 |
| f5 | #F5 | #F5 |  | #F5 | #F5 | #F5 |
| f6 | #F6 | #F6 |  | #F6 | #F6 | #F6 |
| f7 | #F7 | #F7 |  | #F7 | #F7 | #F7 |
| f8 | #F8 | #F8 |  | #F8 | #F8 | #F8 |
| BLUE (f9) | #F9 | #F9 |  | #F9 | #F9 | #F9 |
| YELLOW (f10) | #FA | #FA |  | #FA | #FA | #FA |
| GREEN tick (f11) | #FB | #FB |  | #FB | #FB | #FB |
| RED cross (f12) | #FC | #FC |  | #FC | #FC | #FC |

| ` | ` | ¬ | #D0 (acc) | ` | ` | #80 (typ) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | ! | #D1 (acc) | 1 | 1 | #AD (typ) |
| 2 | 2 | " | #D2 (acc) | 2 | #82 (typ) | #81 (typ) |
| 3 | 3 | £ (#9C) | #D3 (acc) | 3 | 3 | 3 |
| 4 | 4 | $ | #D4 (acc) | 4 | 4 | 4 |
| 5 | 5 | % | #D5 (acc) | 5 | 5 | 5 |
| 6 | 6 | ^ | #D6 (acc) | 6 | 6 | 6 |
| 7 | 7 | & | #D7 (acc) | 7 | 7 | 7 |
| 8 | 8 | * | #D8 (acc) | 8 | 8 | 8 |
| 9 | 9 | ( | #D9 (acc) | 9 | 9 | 9 |
| 0 | 0 | ) | #DA (acc) | 0 | #9B (typ) | #9D (typ) |
| - | - | _ | #DB (acc) | - | #A0 (typ) | - |
| = | = | + | #DC (acc) | = | = | = |
| Backspace | #08 | #08 | #08 | #08 | #08 | #08 |

| Tab | #09 | #09 | #09 | #09 | #09 | #09 |
| --- | --- | --- | --- | --- | --- | --- |
| Q | q | Q | q | q | q | #AB (typ) |
| W | w | W | w | w | w | #AC (typ) |
| E | e | E | e | e | e | #AA (typ) |
| R | r | R | r | r | r | #A9 (typ) |
| T | t | T | t | t | t | #96 (typ) |
| Y | y | Y | y | y | #86 (typ) | #A5 (typ) |
| U | u | U | u | u | u |  |
| I | i | I | i | i | i |  |
| O | o | O | o | o | #94 (typ) | #93 (typ) |
| P | p | P | p | p | p | #9A (typ) |
| [ | [ | { | #DE (acc) | [ | [ | #A6 (typ) |
| ] | ] | } | #DF (acc) | ] | ] | #A7 (typ) |
| RETURN | #0D | #0D | #0D | #0D | #0D | #0D |

| A | a | A | a | a | #92 (typ) | #91 (typ) |
| --- | --- | --- | --- | --- | --- | --- |
| S | s | S | s | s | s | #8B (typ) |
| D | d | D | d | d | d | #89 (typ) |
| F | f | F | f | f | f |  |
| G | g | G | g | g | g | #88 (typ) |
| H | h | H | h | h | h | #8A (typ) |
| J | j | J | j | j | j | #83 (typ) |
| K | k | K | k | k | k | #84 (typ) |
| L | l | L | l | l | l | #85 (typ) |
| ; | ; | : | ; | ; | ; | ; |
| ' | ' | @ | ' | ' | ' | #87 (typ) |
| # | # | ~ | #DD (acc) | # | # | #A3 (typ) |

| \ | \ | ¦ | \ | \ | \ | #95 (typ) |
| --- | --- | --- | --- | --- | --- | --- |
| Z | z | Z | z | z | z | #98 (typ) |
| X | x | X | x | x | x | #99 (typ) |
| C | c | C | c | c | c | #97 (typ) |
| V | v | V | v | v | v | v |
| B | b | B | b | b | b | #90 (typ) |
| N | n | N | n | n | n | #A4 (typ) |
| M | m | M | m | m | m | m |
| , | , | < | , | , | , | #AE (typ) |
| . | . | > | . | . | . | #AF (typ) |
| / | / | ? | / | / | / | #A8 (typ) |
| SPACE | #20 | #20 | #20 | #20 | #20 | #20 |

| PrintScrn | #EA | #EA | #EA | #EA | #EA | #EA |
| --- | --- | --- | --- | --- | --- | --- |
| UNDO (ScrollLock) | #EB | #EB | #EB | #EB | #EB | #EB |
| HELP (Pause) | #EC | #EC | #EC | #EC | #EC | #EC |

| Insert | #E4 | #E4 | #E4 | #E4 | #E4 | #E4 |
| --- | --- | --- | --- | --- | --- | --- |
| Delete | #E5 | #E5 | #E5 | #E5 | #E5 | #9F (typ) |
| Home | #E6 | #E6 | #E6 | #E6 | #E6 | #E6 |
| End | #E7 | #E7 | #E7 | #E7 | #E7 | #E7 |
| PageUp | #E8 | #E8 | #E8 | #E8 | #E8 | #E8 |
| PageDown | #E9 | #E9 | #E9 | #E9 | #E9 | #E9 |

| Up | #E0 | #E0 | #8C (typ) | #E0 | #E0 | #E0 |
| --- | --- | --- | --- | --- | --- | --- |
| Down | #E1 | #E1 | #8D (typ) | #E1 | #E1 | #E1 |
| Left | #E2 | #E2 | #8E (typ) | #E2 | #E2 | #E2 |
| Right | #E3 | #E3 | #8F (typ) | #E3 | #E3 | #E3 |

| Keypad . | . | . | . | . | #A2 (typ) | . |
| --- | --- | --- | --- | --- | --- | --- |
| Keypad 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Keypad 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Keypad 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Keypad 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Keypad 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Keypad 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Keypad 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Keypad 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Keypad 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| Keypad 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| Keypad % | % | % | % | % | % | % |
| Keypad DIVIDE | #A1 (typ) | #A1 (typ) | #A1 (typ) | #A1 (typ) | #A1 (typ) | #A1 (typ) |
| Keypad TIMES | #9E (typ) | #9E (typ) | #9E (typ) | #9E (typ) | #9E (typ) | #9E (typ) |
| Keypad MINUS | #A0 (typ) | #A0 (typ) | #A0 (typ) | #A0 (typ) | #A0 (typ) | #A0 (typ) |
| Keypad + | + | + | + | + | + | + |
| ENTER | #0D | #0D | #0D | #0D | #0D | #0D |

# Appendix : The Text System

This appendix describes Anne's Typography and Font system in detail.  The character set described here is used for all data entry and system dialogues in Anne applications.  In a few situations (eg entering file-names), it will be possible to type only those characters which are printed on the key-tops, but when typing text into a program, the full character-set shown below will be available.

## Character Set

The diagram below shows the printable characters available in an Anne font. Character codes 00-1Fh and E0-FFh are reserved for system control codes and other non-printable characters.  Codes D0-DFh are accents, which can only be used in combination with another character.

All character codes in the range B0-CFh are available as printable characters, though they are left blank in the standard release character set: we can eventually provide an add-on utility to design custom characters for the vacant slots.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  | Sp | 0 | @ | P | ‘ | p | “ | ß | — |  |  | Grave |  |  |
| 1 |  |  | ! | 1 | A | Q | a | q | ” | Æ | ¸ |  |  | Acute |  |  |
| 2 |  |  | " | 2 | B | R | b | r | „ | æ | · |  |  | Umlaut |  |  |
| 3 |  |  | # | 3 | C | S | c | s | · | Œ | ° |  |  | Tilde |  |  |
| 4 |  |  | $ | 4 | D | T | d | t |  | œ |  |  |  | Hacek |  |  |
| 5 |  |  | % | 5 | E | U | e | u |  | c/o |  |  |  | Ring |  |  |
| 6 |  |  | & | 6 | F | V | f | v | ¥ | ™ | † |  |  | C/flex |  |  |
| 7 |  |  | ' | 7 | G | W | g | w | ¤ | © | ‡ |  |  | O/Dot |  |  |
| 8 |  |  | ( | 8 | H | X | h | x | § | µ § | ¿ |  |  | Macron |  |  |
| 9 |  |  | ) | 9 | I | Y | i | y | ¨ |  | ® |  |  | Breve |  |  |
| A |  |  | * | : | J | Z | j | z | © | ¢ | ¾ |  |  | D/acute |  |  |
| B |  |  | + | ; | K | [ | k | { | ª | ø | ½ |  |  | Apostr. |  |  |
| C |  |  | , | < | L | \ | l | ¦ |  | £ | ¼ |  |  | U/Dot |  |  |
| D |  |  | - | = | M | ] | m | } | ¯ | Ø | ¡ |  |  | Comma |  |  |
| E |  |  | . | > | N | ^ | n | ~ | ¬ | × | « |  |  | Cedilla |  |  |
| F |  |  | / | ? | O | _ | o | ’ | ® | ± | » |  |  | Ogonek |  |  |

### Notes:

83-85h are "Bullets", centred vertically in the Cap height.

98-99h are musical Flat and Natural symbols.

9E-A3h are maths symbols (Multiply, +/-, Minus, Divide, mid-height Decimal Point, and Degrees).

7Fh (see Quotes below) is typed using Shift+`.

Quotes: there are "flat" single (27h) and double-quotes (22h) AND open/close single (60h and 7Fh) and double-quotes (80-81h), plus French Guillemets (AE-AFh) and the German open-quote (82h).  The "open single" and "close single" characters will be printed on the top left alpha-key above Tab, for Normal and Shifted states repectively.  This key is also for Open Double (Ctrl+`) and Close Double (Shift+Ctrl+`) quotes.

## Accents

Accents are single bytes which precede the character to which they are to be added.  Their height above the baseline is automatically adjusted to match that of the character.

D0-DFh are the same as the accents used in MD3, comprising twelve "over" and four "under" accents.  As in MD3, the vertical position of the over-accents varies with the height of the character, and the horizontal position is automatically centred over the character (except for the Apostrophe accent).  When an "over" accent is added to an "i" or "j", the dot is shaved off the top of the character before the accent is added.

The Under accents are automatically centred below the character, but their vertical position does not change.  (Adding an Under accent to a character with a descender will produce strange results.)

In order to keep the accent positioning system as simple as possible, we will limit the range of characters to which an accent can be added.  The range will comprise Space (20h), the capital and lower alpha characters (41-5Ah and 61-7Ah inclusive), the diphthongs (91-94h), and the O-slash (9Bh and 9Dh).

### Control Codes ... Non-printing Characters

In addition to the printable characters, the following control symbols may need to be displayed, and will presumably need to have control codes assigned to them.  Note that the actual designs of these characters may be different to those illustrated...

¶	Paragraph (or "¿" for Return, if that's what's printed on the key)

===	Hard Page Break (double-line across WP display cf  Word)

-	Optional (soft) hyphen (reserved for future release)

®	Tab

.	Space (when view option selected)

_	Non-break character indicator

### Control Codes ... Character Style Codes

There are 7 style control codes which operate as toggles...

Bold, Underline, Italic, Superscript, Subscript, Non-prop Number-Spacing.

("Non-prop Number Spacing" forces 0-9 numeric characters to use a fixed pitch, without affecting other characters.  This allows properly lined-up columns of figures without needing to design or use fixed-pitch fonts.)

## Keyboard Typing

In general, command shortcuts are implemented using Ctrl+Alpha keys, Menus are accessed using Tasks+Alpha keys, and Phrases are Stored using Shift+Ctrl+F1-8 and played using Ctrl+F1-8.  Characters which do not appear on key-tops can be selected using a mouse via the "Insert Character" command in the WP, but can also be typed at he keyboard using Shift+Ctrl+Alpha keys as shown in the table below.  There are also a few Ctrl+ combinations, but not on the Alpha keys.

The remaining 48 combinations of Tasks+Ctrl+Alpha keys can be assigned to the user-definable characters which are blank in the release character sets.

The table below shows the key-combinations used to type non-ASCII characters - Accents should be typed before the character to which they are to be added.  Note that the key referred to in the table as ` is the top-left key above TAB.

|  | 80-F |  | 90-F |  | A0-F |  | D0-F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | “ | Ctrl+` | ß | Sh+Ctrl+B | — | — ** Ctrl+- | Grave | Tasks+` |
| 1 | ” | Sh+Ctrl+` | Æ | Sh+Ctrl+9 | ¸ | ¸ ** | Acute | Tasks+1 |
| 2 | „ | Ctrl+2 | æ | Ctrl+9 | · | Ctrl+. ** | Umlaut | Tasks+2 |
| 3 | · | Sh+Ctrl+J | Œ | Sh+Ctrl+8 | ° | Sh+Ctrl+# | Tilde | Tasks+3 |
| 4 |  | Sh+Ctrl+K | œ | Ctrl+8 |  | Sh+Ctrl+Red | Hacek | Tasks+4 |
| 5 |  | Sh+Ctrl+L | c/o | Sh+Ctrl+\ |  | Sh+Ctrl+Green | Ring | Tasks+5 |
| 6 | ¥ | Sh+Ctrl+Y | ™ | Sh+Ctrl+T | † | Sh+Ctrl+[ | C/flex | Tasks+6 |
| 7 | ¤ | Sh+Ctrl+; | © | Sh+Ctrl+C | ‡ | Sh+Ctrl+] | O/Dot | Tasks+7 |
| 8 | § | Sh+Ctrl+G | µ § | Sh+Ctrl+Z | ¿ | Sh+Ctrl+/ | Macron | Tasks+8 |
| 9 | ¨ | Sh+Ctrl+D |  | Sh+Ctrl+X | ® | Sh+Ctrl+R | Breve | Tasks+9 |
| A | © | Sh+Ctrl+H | ¢ | Sh+Ctrl+P | ¾ | Sh+Ctrl+E | D/acute | Tasks+0 |
| B | ª | Sh+Ctrl+S | ø | Ctrl+0 | ½ | Sh+Ctrl+Q | Apostr. | Tasks+- |
| C |  | Tasks+ | £ | £ | ¼ | Sh+Ctrl+W | U/Dot | Tasks+= |
| D | ¯ | Tasks+¯ | Ø | Sh+Ctrl+0 | ¡ | Sh+Ctrl+1 | Comma | Tasks+# |
| E | ¬ | Tasks+¬ | × | × ** | « | Sh+Ctrl+, | Cedilla | Tasks+[ |
| F | ® | Tasks+® | ± | Sh+Ctrl+Del* | » | Sh+Ctrl+. | Ogonek | Tasks+] |

*   Del key above cursor-keys - this is used for +/- in the Calculator.

**  There are dedicated keypad keys for ×, ¸, —, % and Point, in addition to the + key: these all generate appropriate characters in the WP.  Ctrl+Point gives the mid-height style of point.  Enter emulates Return.  The Minus character also functions as a dash or long hyphen (—), and can be typed using the keypad "minus" key, or Ctrl+Hyphen.